SUPPLEMENTAL MEASUREMENT S10B

MATERIALS AND METHODS

S10 participated twice within three weeks to examine reproducibility (Table 1). The two measurements are labeled with S10A and S10B. To assess reproducibility in-between the measurements of S10A and S10B, only slices that are in the FOV of both measurements were analyzed.  To determine if cold-activated regions appear at comparable anatomical sites, the FF maps of S10A and S10B showing assumed BAT depots were compared with the help of a board-certified radiologist (M.T.F., 6 years of experience in MRI).

RESULTS

Temperature Measurements

Figure 1 (a) shows a direct comparison of the temperature profile of S10A and S10B. During the initial and the cooling phase, outlet water temperature was higher than the inlet water temperature for S10A and S10B (Table 2) what resulted in positive corrected heat transfer values Q’intitial,S10A,B and Q’cool,S10A,B during each phase (Figure 2). Detailed calculations on heat transfer determination are delineated in the Supplemental Heat Transfer Determination. During warming, reactions to temperature changes were different in between S10A and S10B. For S10A, water inlet temperature was higher than the outlet temperature, resulting in a negative heat transfer Q’warm,S10A. In comparison, Q’warm,S10B was determined to be positive for S10B.

Body temperatures stayed constant at TBody,S10A = (36.9 ± 0.5)°C and TBody,S10B = (38.2 ± 0.2)°C. Back skin temperature decreases are illustrated in Figure 1(b). Comparing the maxima, back skin temperature decreased about (2.3 ± 0.7)°C more for S10A.

Fat Fraction Data Analysis

Figure 3 shows the FF results for the evaluated BAT, SAT and muscle tissue ROIs. Detailed quantified results for the ROIs are given in Table 3.

In both measurements, significant mean BAT FF decreases during the initial (pinitial < 0.001) and the cooling phase (pcool < 0.001) were observed for S10A and S10B.  During warming, BAT FF decrease continued significantly (pwarm,S10A,B < 0.001).

Neither in SAT nor in muscle tissue a FF change was observed for S10A and S10B during the whole measurement time (pSAT,muscle < 0.001). Individual p-values are delineated in Table 4. The evaluated BAT volume of 1.07 ml for S10B is smaller compared to 1.50 ml or S10A. Figure 4 illustrates for BAT, SAT and muscle tissue ROIs the determined individual slopes (slopeinitial/cool/warm, slopeSAT/muscle), mean FF values as well as the evaluated BAT volume.

For S10A and S10B, correlations between BAT FF decrease and back skin temperature under cold exposure (Figure 5 (a)) were observed. Resultant, negative correlated slopes slopeFFbs (Figure 5(b)) as well as correlation coefficients of │rFFbs│ > 0.5 (Table 4) were determined for both measurements.

DISCUSSION

The repetition measurement B of S10 showed the same qualitative results as measurement A. Comparing the anatomical slices in the same FOV of S10A and S10B, cold-activated supposed BAT depots could be found in both measurements.

Temperature results were comparable for S10A and S10B aside from the stronger back skin temperature decrease of S10A and the negative corrected heat transfer Q’warm,S10A, indicating that the subject freezed more during measurement A. The evaluated smaller volume during measurement B indicates a variation on the amount of cold-activated BAT.

These two results outline the limitation that more volunteers need to participate twice. Detailed statistical validation would be possible and more correlations could be evaluated.   This includes investigating if there are correlations between the amount of cold-activated BAT and gender, age and BMI. Repeated measurements during different seasons would help to determine potential correlations between outdoor temperature and the mentioned parameters. However, even if only one measurement was repeated, results of S10A and S10B show determined cold-activated regions, non-changing FF values in SAT and muscle tissue as well as a correlation between FF decrease in BAT and back skin temperature, emphasizing the potential of continuing measurements.

TABLES

Table 1 Gender, height, weight and body mass index (BMI) and age of the study participants.

Table 1

| Subject | Gender | Height [m] | Weight [kg] | BMI | Age [yrs] |
| --- | --- | --- | --- | --- | --- |
| S10 | f | 1.80 | 75 | 23.2 | 27 |

Table 2 Temperature differences (∆T) of the inlet and outlet water temperature of the cooling vest for initial, cooling and warming phase compared for S10A and S10B. Values of heat transfers Qinitial, Qcool and Qwarm resulting from the corresponding particular ∆T quantify the heat transfer of the cooling vest with the volunteers and the environment. Q’initial, Q’cool and Q’warm are the corrected heat transfer values quantifying the heat transfer between cooling vest and volunteers minus the heat transfer part of the environment, as derived in the baseline measurement.. Values Q’initial, Q’cool and Q’warm are illustrated in Figure 2.

Table 2

| ## Subject | ∆Tinitial | ## Qinital ## Q’initial | ∆Tcool | ## Qcool ## Q’cool | ∆Twarm | ## Qwarm ## Q’warm |
| --- | --- | --- | --- | --- | --- | --- |
|  | [°C] | ## [W] | [°C] | ## [W] | [°C] | ## [W] |
| ## Phantom | ## 0.5 ± 0.1 | ## 8 ± 3 ## --- | ## 0.8 ± 0.1 | ## 24 ± 4 ## --- | ## 0.2 ± 0.1 | ## -6 ± 3 ## --- |
| ## S10A | ## 0.5 ± 0.1 | ## 27 ± 1 ## 19 ± 4 | ## 1.1 ± 0.1 | ## 56 ± 1 ## 32 ± 4 | ## -1.4 ± 0.1 | ## -70 ± 1 ## -64± 4 |
| ## S10B | ## 0.7 ± 0.1 | ## 37 ± 1 ## 29 ± 4 | ## 1.2 ± 0.1 | ## 60 ± 1 ## 36 ± 4 | ## -0.1 ± 0.1 | ## -1 ± 1 ## 6 ± 3 |

Table 3 Results for the evaluated regions of interest (ROIs) brown adipose tissue (BAT), subcutaneous adipose tissue (SAT) and muscle tissue. The mean fat fraction (mean ± standard deviation in ROIs of S10A,B) of both BAT depots at the beginning of the measurement (FFstart), before the cooling phase (FFbase) and at the end of the cooling phase (FFcool) as well as at the end of the warming phase (FFend) are compared. Occurring FF changes in the total volumes of the two evaluated BAT ROIs during the three temperatures phases were estimated by the slope obtained from a linear fit (slope ± fiterror). Results are illustrated in Figure 4.

Table 3

|  | Interscapular BAT |  |  |  |  |  |  |  | SAT |  |  | Muscle Tissue |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FFstart [%] | FFbase [%] | FFcool [%] | FFend [%] | Slopeinitial [% / h] | Slopecool [% / h] | Slopewarm [% / h] | Volume [ml] |  | FFSAT [%] | SlopeSAT [% / h] |  | FFmuscle [%] | Slopemuscle [% / h] |  |
| S10A | 71±6 | 71±6 | 69±7 | 69±7 | -1.9±2.5 | -1.4±0.4 | -2.8±3.3 | 1.50 |  | 83±1 | 0.1±0.1 |  | 10±1 | 0.1±0.1 |  |
| S10B | 72±5 | 71±7 | 69±6 | 70±8 | -2.0±2.4 | -1.8±0.5 | 1.2±2.4 | 1.07 |  | 83±1 | 0.1±0.1 |  | 10±1 | 0.3±0.1 |  |

Table 4 Statistical results for S10A and S10B. P-values for the determination of fat fraction (FF) changes in the evaluated brown adipose tissue (BAT) regions of interest (ROIs) as well as in the ROIs of subcutaneous (SAT) and muscle tissue were determined with a paired t-test. For S10A and S10B, FF changes in BAT were significant during each phase in both measurements. In SAT and muscle tissue, no significant FF changes were observed. Correlations between FF decrease in BAT during the cooling phase and back skin temperature decrease of S10A and S10B showed a negative correlated slope (slopeFFbs) and correlation coefficients were observed to be │rFFbs│> 0.5 for both measurements.

Table 4

|  | p-values |  |  |  |  | Correlations |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | pinitial, BAT | pcool,BAT | pwarm,BAT | pSAT | pmuscle | slopeFFbs [% / °C] | │rFFbs│ |  |
| S10A | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | -0.47±0.10 | 0.77 |  |
| S10B | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | -0.90±0.14 | 0.86 |  |

FIGURE LEGENDS

Figure 1

(a) Comparison of temperature profiles for S10A and S10B. Colors are consistent to the main document. For both measurements, the body temperature measured under the armpit (blue) stayed constant. Outlet water temperature (yellow) was higher compared to the inlet water temperature (green) during the initial and cooling phase for S10A and S10B. During the warming phase of S10A, inlet water temperature was higher, contrarious to results of S10B, showing a lower inlet water temperature compared to the outlet temperature during warming. (b) Illustration of back skin temperature decreases of S10A,B. Start temperatures at Tstart = 0 min were shifted to zero and averaged in a span of 111 data points to filter noise. Temperature decrease steps from -1°C to -7°C are marked with horizontal dashed lines. Back skin temperatures show a similar curve progression during each temperature phase. During the measurement of S10A, back skin temperature decreased nearly about 7°C, compared to measurement B showing a decrease of nearly 5°C.

Figure 2

Illustration of quantified corrected heat transfer values Q’initial, Q’cool and Q’warm between S10A,B and the cooling vest during the initial (green), cooling (blue) and warming (red) phase. Heat transfer parts of the environment with the cooling vest were subtracted with the help of a baseline measurement with an isolated phantom. Quantified heat transfer values are given in Table 2.

Figure 3

Fat fraction (FF) values over time of the evaluated regions of interest (ROIs) in brown adipose tissue (BAT, red), subcutaneous adipose tissue (SAT, blue) and muscle tissue (yellow) compared for S10A,B. FF changes in BAT over time were estimated by applying a linear fit for each phase (exemplarily shown for the cooling phase). Significant FF changes were observed during each phase. FF values in SAT and muscle tissue did not change during the experiment (p < 0.001). Individual data are given in Table 2. Differentiation of the three temperature phases (initial, cooling, warming) is the same as in Figure 1.

Figure 4

Bar plot illustration of quantified fat fraction (FF) decreases over time estimated by a linear fit in brown adipose tissue (BAT) depots during the initial (a), cooling (b) and warming (c) phase compared for S10A,B. Corresponding mean FF values (mean ± standard deviation) in the evaluated BAT volume (d) at the beginning of the measurement (FFstart), at the phase transition to the cooling phase (FFbase) and to the warming phase (FFcool) as well as at the end of the measurement (FFend) are illustrated in (e). Mean FF values in subcutaneous adipose tissue (SAT) and muscle tissue (f) and the corresponding linear slopes during the whole measurement time (g) are also delineated. Quantified values are delineated in Table 2.

Figure 5

(a) Fat fraction (FF) values in evaluated brown adipose tissue (BAT) regions of interest (ROIs) are plotted against corresponding back skin temperatures (averaged in a span of 60 data points each 5 min) for the initial (green), cooling (blue) and warming (red) phase. Correlated slopes (slopeFFbs) were determined by a linear fit (black) on the significant FF changes (p < 0.001) during the cooling phase. While back skin temperatures decreased, a FF decrease in BAT could also be observed. (b) Bar plot illustration of resulting negative values slopeFFbs. Correlations │rFFbs│> 0.5 were observed for S10A and S10B. Quantified data is delineated in Table 4.