## SUPPLEMENTAL HEAT TRANSFER DETERMINATION

## MATERIALS AND METHODS

The identical measurement setup was performed with an isolated thorax phantom to quantify the heat transfer (Q = cW × fw × ∆T; specific heat capacity of water: cW = 4182 J/(kg×K) and experimentally measured water mass flow rate: fw = 0.012 kg/s) between the cooling vest and its environment (ambient air and scanner table). The thorax phantom was built in-house with acrylic glass and had the shape of an elliptical cylinder (semiaxes: 0.19 m and 0.13 m, height: 0.55 m). Insulation material LOGOCLIC® XPS Maxima (LOGOCLIC®, Großenhain, Germany, heat transfer resistance: 0.171m²K/W) with 4 mm thickness was wrapped two times around the phantom. Temperatures were measured at the inlet and the outlet of the cooling vest and between the phantom surface and the insulation. For each temperature phase, heat transfer values Qinitial,phantom, Qcool,phantom and Qwarm,phantom were determined. During the measurements of the volunteers, heat transfer values Qinitial, Qcool and Qwarm between the volunteers and the cooling vest and environment were also determined for each temperature phase.

To consider only the heat transfer of the volunteers into the water circuit, heat transfer parts between the cooling vest and the environment were excluded. Therefore, the heat transfer parts of the environment into the water circuit were subtracted from the mean values Qinitial, Qcool and Qwarm and corrected heat transfer values Q’initial and Q’cool and Q’warm were determined.

RESULTS

The temperature differences ∆Tinital and ∆Tcool  indicate a heat transfer during the initial phase and during the cooling phase from the volunteers and the environment into the water circuit. Heat transfer quantification results in mean heat transfer values of Qinitial = (35 ± 5) W during the initial phase and of Qcool = (63 ± 4) W during the cooling phase.

This baseline measurement showed a heat transfer of Qinitial,phantom = (8 ± 3) W during the initial phase and of Qcool,phantom = (24 ± 4) W during the cooling phase, including the subtraction of the heat transfer resulting from the temperature difference between the phantom surface and the insulation of 4.3 K.

Thus, the corrected mean heat transfer values from the volunteers into the water circuit are Q’initial = (27 ± 5) W and Q’cool = (39 ± 4) W. During the warming phase, the outlet water temperature was lower than the inlet water temperature for S01, S02, S04 and S10A with a mean temperature difference of ∆Twarm  = (-0.3 ± 0.5)°C. For those subjects, this change of sign resulted in a reversed mean heat transfer of Qwarm- = (-32 ± 29) W of the water circuit. For the subjects showing a higher outlet temperature during the warming phase, mean heat transfer value was Qwarm+ = (3 ± 2) W.  After correction, heat transfer values during the warming phase are Qwarm' - = (-26 ± 29) W and Qwarm' + = (9 ± 2) W. For all subjects, mean heat transfer values during the warming phase are Qwarm = (-13 ± 26) W and Q’warm = (-7 ± 4) W. Figure 1 illustrates the corrected heat transfer results. No data was available from S05 since this measurement was stopped by the volunteer after 105 min before time point Tend was reached. Quantified heat transfer data of all subjects are given in Table 1.

DISCUSSION

The heat transfer values Q’initial and Q’cool were in a similar range for all volunteers. For each subject, mean outlet temperatures at the cooling vest were higher than the mean water temperatures at the inlet of the vest, resulting in a positive sign of the corrected values Q’initial and Q’cool. During the warming phase, individual reactions of the human body to the different water temperature phases were observed, resulting in varying signs of Q’warm and a large standard deviation of the corresponding mean value.

TABLES

Table 1 Temperature differences (∆T) of the inlet and outlet water temperature of the cooling vest for initial, cooling and warming phase. Values of heat transfers Qinitial, Qcool and Qwarm resulting from the corresponding particular ∆T quantify the heat transfer of the cooling vest with the volunteers and the environment. Q’initial, Q’cool and Q’warm are the corrected heat transfer values quantifying the heat transfer between cooling vest and volunteers minus the heat transfer part of the environment, as derived in the baseline measurement. Values are averaged and given as mean ± standard deviation determined of the results of S01-S10A. Values Q’initial, Q’cool and Q’warm are illustrated in Figure 1.

Table 1

| ## Subject | ∆Tinitial | ## Qinital ## Q’initial | ∆Tcool | ## Qcool ## Q’cool | ∆Twarm | ## Qwarm ## Q’warm |
| --- | --- | --- | --- | --- | --- | --- |
|  | [°C] | ## [W] | [°C] | ## [W] | [°C] | ## [W] |
| ## Phantom | ## 0.5 ± 0.1 | ## 8 ± 3 ## --- | ## 0.8 ± 0.1 | ## 24 ± 4 ## --- | ## 0.2 ± 0.1 | ## -6 ± 3 ## --- |
| ## S01 | ## 0.8 ± 0.1 | ## 39 ± 1 ## 31 ± 4 | ## 1.2 ± 0.1 | ## 61 ± 1 ## 38 ± 4 | ## -0.1 ± 0.1 | ## -7 ± 1 ## -1 ± 3 |
| ## S02 | ## 0.6 ± 0.1 | ## 31 ± 1 ## 23 ± 4 | ## 1.3 ± 0.1 | ## 64 ± 1 ## 40 ± 4 | ## -0.2 ± 0.1 | ## -12 ± 1 ## -6 ± 4 |
| ## S03 | ## 0.8 ± 0.1 | ## 38 ± 1 ## 30 ± 4 | ## 1.3 ± 0.1 | ## 67 ± 1 ## 44 ± 4 | ## 0.1 ± 0.1 | ## 2 ± 1 ## 8 ± 4 |
| ## S04 | ## 0.6 ± 0.1 | ## 27 ± 1 ## 19 ± 4 | ## 1.2 ± 0.1 | ## 61 ± 1 ## 37 ± 4 | ## -0.8 ± 0.1 | ## -40 ± 5 ## -34 ± 6 |
| ## S05 | ## 0.6 ± 0.1 | ## 31 ± 1 ## 23 ± 4 | ## 1.2 ± 0.1 | ## 58 ± 1 ## 34 ± 4 | ## n.a | ## n.a ## n.a |
| ## S06 | ## 0.8 ± 0.1 | ## 41 ± 1 ## 33 ± 4 | ## 1.4 ± 0.1 | ## 70 ± 1 ## 46 ± 4 | ## -0.1 ± 0.1 | ## 1 ± 1 ## 7 ± 4 |
| ## S07 | ## 0.8 ± 0.1 | ## 39 ± 1 ## 31 ± 4 | ## 1.3 ± 0.1 | ## 64 ± 1 ## 40 ± 4 | ## -0.1 ± 0.1 | ## 3 ± 1 ## 9 ± 4 |
| ## S08 | ## 0.8 ± 0.1 | ## 38 ± 1 ## 30 ± 4 | ## 1.3 ± 0.1 | ## 64 ± 1 ## 40 ± 4 | ## -0.1 ± 0.1 | ## 5 ± 1 ## 11 ± 4 |
| ## S09 | ## 0.7 ± 0.1 | ## 37 ± 1 ## 29 ± 4 | ## 1.3 ± 0.1 | ## 63± 1 ## 39 ± 4 | ## -0.1 ± 0.1 | ## 3 ± 1 ## 9 ± 4 |
| ## S10A | ## 0.5 ± 0.1 | ## 27 ± 1 ## 19 ± 4 | ## 1.1 ± 0.1 | ## 56 ± 1 ## 32 ± 4 | ## -1.4 ± 0.1 | ## -70 ± 1 ## -64± 4 |
| ## mean | ## 0.7 ± 0.1 | ## 35 ± 5 ## 27 ± 5 | ## 1.3 ± 0.1 | ## 63 ± 4 ## 39 ± 4 | ## -0.3 ± 0.5 | ## -13 ± 26 ## -7 ± 4 |

FIGURE LEGENDS

Figure 1

Illustration of quantified corrected heat transfer values Q’initial, Q’cool and Q’warm between the volunteers and the cooling vest for each volunteer during the initial (green), cooling (blue) and warming (red) phase. Heat transfer parts of the environment with the cooling vest were subtracted with the help of a baseline measurement with an isolated phantom. Detailed heat transfer values are given in Table 1.