SUPPLEMENTAL REGISTRATION EVALUATION

MATERIALS AND METHODS

To evaluate the FSFDemons quantitatively and to estimate an error value, transformations were assessed via Point Set Interaction (plugin of MITK).

This evaluation was done by a board-certified radiologist (M.T.F., 6 years of experience in MRI) by placing a set of 5 positions X = {xi | i = 1...5} at specific anatomical landmarks on the target data set (DT) of each volunteer (Figure 1(a)). Landmarks were chosen that typically do not extend one slice and were not affected by motion artifacts.

The positions X’ = {xi’ | i = 1...5} and X’’ = {xi’’ | i = 1...5} of the same landmarks were placed at two unregistered data sets. Therefore, for each volunteer, the third dataset measured at the beginning (DM1), and the third last (DM2), measured at the end of the experiment, were chosen. Using the corresponding transformations calculated by the image registration algorithm, the positions of the sets X’ and X’’ were mapped on the positions of the target data set X. Mapping was performed using MITK.

For all volunteers, the deviation vectors ∆si = xi,M1/2 – xi for all landmarks Li (i=1…5) were calculated. Finally mean deviations ∆si,mean for each landmark were examined.

RESULTS

Figure 1(b) delineates the distributions separately for each landmark Li. The minimum and maximum deviations, the second and third quartile, and the median M are given. Each boxplot shows a space coordinate of the corresponding deviation vector ∆si and is generated out of twenty data points. The mean deviations ∆si,mean scatter around zero in the range [-0.84; 0.49] mm. Also all median values scatter in positive and negative direction M = [-0.60; 0.51] mm.

After mapping the landmarks L1 – L5 on the target data set, limits of variations are in the range of ∆si,mean = [0.23; 0.90] mm, whereof L2 showed the smallest limit of variation with ∆s2 = 0.23 mm ≙ 0.19 pixel and landmark L4 and L5 showed the highest limit of variation with ∆s4 = ∆s5 = 0.90 mm ≙ 0.75 pixel.

Combining the results for all landmarks L1-L5 and focusing on the directions in space (x-,y-,z-components), mean deviation vectors are ∆m = (-0.02; -0.16; -0.23) mm. The upper limit of the distribution resulting from the standard deviations of ∆m is 1.3 pixels.

DISCUSSION

This study was confined to the interscapular depots. The chest region including potential BAT depots in the supraclavicular region was excluded from evaluation to prevent registration errors due to varying breath-hold positions over the long total measurement time of 140 min.

An evaluation of the FSFDemons with the use of point sets was conducted to assess its performance for compensation of breath-hold position offsets between image acquisition (31, 32). Registration of all data sets to a target data set did not provoke a systematic displacement of the landmarks. Taking into account the absolute volume of 1.31 ml of evaluated BAT regions, a relative error for BAT region displacements can be estimated. Assuming a simplified cubical shape, the mean BAT region has a side length of 10.9 mm ≙ 9.1 pixels. A shift using the mean deviations ∆m results in an overlap of 96% of the mean BAT region before and after displacement, which corresponds to a relative error of 4%. Considering the maximum deviations of directions in space, nearly no overlap (2%) exists. The landmarks L2, L3, and L4 are the closest to BAT sites. Even if L4 and L5 showed the highest limits of variation, this displacement is smaller than one pixel side length. Based on this error estimation, the FSFDemons was considered suitable for the evaluated regions. Almost constant FF in SAT and muscle tissue with relatively small errors also indicates good quality of the registration algorithm in the corresponding regions. An extensive segmentation of non-adipose tissue to detect BAT (41) could provide a more detailed assessment of the registration quality. However, exact delineation of tissue types is difficult owing to the heterogeneous mixture of human BAT and WAT and the individual distribution of BAT in adults. This might even result in larger errors in BAT detection.

FIGURE LEGENDS

Figure 1

(a) Landmark positions set by a board-certified radiologist for evaluation of the FSFDemons algorithm: L1: Right acromioclavicular joint, axial orientation; L2: Branching of the left vertebral artery from the subclavian artery, coronal orientation; L3: Left costo-vertebral joint space of the 1st thoracic vertebra, axial orientation; L4: Processus spinosus of the 7th cervical vertebra, sagittal orientation; L5: Left tuberculum majus humeri, sagittal orientation. (b) Deviation vectors ∆si for each landmark. Boxplots show the deviations of the vector components ∆si,x/y/z after mapping the point sets of the unregistered data sets DM1 and DM2 on the point sets of the target data set DT. The minimum and maximum deviation and the second and third quartile of the deviations are drawn, separated by the particular median. 1.2 mm deviation corresponds to 1 pixel length.