Supplementary Data to

Intensify production, transform biomass to energy and novel goods and protect soils in Europe – a vision how to mobilize marginal lands

Schröder P1*, Beckers B2, Daniels S2, Gnädinger F1, Maestri E3, Marmiroli, N3, Mench M4, Millan R5, Obermeier MM1, Oustriere N4, Persson T6, Poschenrieder C7, Rineau F2, Rutkowska B8, Schmid T5, Szulc W8, Witters N2, Sæbø A6

*corresponding author (mail: peter.schroeder@helmholtz-muenchen.de)

1Helmholtz Zentrum Muenchen, German Research Center for Environmental Health, GmbH, COMI, Ingolstädter Landstrasse 1, D-85764 Neuherberg, Germany.

2Hasselt University, Agoralaan Gebouw D - B-3590 Diepenbeek, Belgium

3University of Parma, Department of Chemistry, Life Sci. Environm. Sustainability - Parco Area delle Scienze 11A, I-43124 Parma, Italy

4UMR BIOGECO INRA 1202, Bordeaux University, France

5CIEMAT - Departamento de Medio Ambiente, Avenida Complutense 40, E-28040 Madrid, Spain

6NIBIO - Norwegian Institute of Bioeconomy Research, NO-1431 Klepp, Norway

7Martl-Hof, Hochwiesweg 6, D-83703 Ostin, Germany

8Warsaw University of Life Sciences - SGGW, 02-787 Warsaw, Poland

In situ techniques

Relatively unrestricted

Widespread

Low to medium

Deep within site

Ex situ techniques

Less than a year free

Localised

Medium to high

Relatively near surface

Technology transition

time

contamination

concentration

depth

Supplementary Fig. 1 Factors involved in the choice of the remediation technology

Supplementary Table 1 Composting conditions required to ensure waste hygienization.

According to US Environmental Protection Agency regulations (EPA, 2002), maintaining a minimum temperature of the composting mass of 55° C for 3 days (aerated static pile or in-vessel) or 15 days with 5 turns is recommended to meet the regulatory requirements of class A fertilizers, and a minimum of 40° C for 5 days - during which temperature should exceed 55° C for at least 4 h - to meet class B fertilizer requirements.

| Method of composting | Temperaturę [oC] | Turn number of composting mass | Holding time (weeks) |
| --- | --- | --- | --- |
| Composting in pile | 55 | 2 | 5 |
| Composting in pile | 65-70 | 1 | 2 |
| Composting in reactors | 60 | 1 | - |

Essential factors:

Among factors influencing the composting process, the more important are (a) the C/N ratio - this relationship depends on the dynamics of microbial processes. The optimal ratio is 25-30. High C/N ratios make this process very slow as there is an excess of degradable substrate for the microorganisms. In the low C/N ratios microbiological processes stop, which may lead to leaching nitrogen from composting mass; (b) pH: optimum values are between 5.5 and 8.0. Usually pH is not important for composting. However it becomes relevant in controlling N-losses by ammonia volatilization, especially at high pH, e.g. >7.5; (c) aeration is an important factor for composting, with the optimum O2 concentration between 15% and 20%. Turning the compost pile provides air circulation, temperature maintenance and proper development of aerobic microorganisms which determine the speed of the composting process; and (d) moisture: The optimum water content for composting varies between 50–60%. Moisture contents higher than 60% inhibit the composting process due to low oxygen concentration, on the other hand, if moisture is too low, the composting process will be hampered.

Supplementary Table 2: Examples of field trials to demonstrate the phytomanagement of trace element contaminated soils

| Plant species | Trace element | Remediation type | Reference |
| --- | --- | --- | --- |
| Berkheya coddii | Ni, Co | Phytoextraction | Keeling et al., 2003 |
| Alyssum murale |  |  | Bani et al., 2017 |
| Geissois pruinosa Brongn. & Gris | Ni | Hyperaccumulators Phytoextraction | Losfeld et al., 2015 |
| Psychotria douarrei | Ni |  | Grison et al., 2013 |
| Grevillea exul Lindl. | Mn | Hyperaccumulator Phytoextraction | Losfeld et al., 2015, Escande et al., 2015 |
| Betula celtiberica | As | Bioaugmented phytoextraction | Mesa et al., 2017 |
| Pityrogramma calomelanos var. Austroamericana, Pteris vittata | As | Hyperaccumulators, phytoextraction | Niazi et al., 2012 |
| Rosa multiflora Thunb. ex Murray, Sida hermaphrodita Rusby | Cr, Ni, Cu, Zn, and Cd | phytoextraction | Antonkiewicz et al., 2017 |
| Iberis intermedia | Tl | Hyperaccumulator Phytoextraction | Grison et al., 2015b |
| Anthyllis vulneraria subsp. carpatica | Zn | Hyperaccumulator Phytoextraction | Frérot et al., 2006; Mahieu et al., 2013 ; Grison, 2015; Grison et al., 2015a |
| Noccaea caerulescens | Zn | Hyperaccumulator Phytoextraction | Losfeld et al., 2012; Grison, 2015 |
| Noccaea caerulescens, Noccaea praecox, Arabidopsis halleri | Zn, Cd, Pb | Hyperaccumulators Phytoextraction | Tlustoš et al., 2016 |
| Sedum plumbizincicola | Zn, Cd | Hyperaccumulator intercropping with maize, phytoextraction | Deng et al., 2016 |
| Sedum plumbizincicola intercropped with Medicago sativa | PCB, Cd, Cu | Hyperaccumulator intercropping with alfalfa, phytoextraction, rhizodegradation | Wu et al., 2012 |
| Sedum alfredii co-planted with Alocasia marorrhiza | Metals, PAH | Hyperaccumulator co-planting with elephant ear, phytoextraction, rhizodegradation | Qiu et al., 2014 |
| poplar cultivar ‘SKADO’ (Populus maximowiczii x P. trichocarpa) | Zn, Cd | Phytoextraction | Bert et al., 2017 |
| Salix ‘Tora’ | Cd, Zn | Phytoextraction | Delplanque et al., 2013 |
| Salix viminalis and Salix alba x alba clones | Zn, Cd | Phytoextraction | Janssen et al., 2015 |
| Willows, poplars | Zn, Cd | Phytoextraction | Kidd et al., 2015 |
| Willows, poplars | Zn, Cd, Pb | Phytoextraction | Kacalkova et al., 2015; Kubatova et al., 2016 |
| Salix schwerinii x Salix viminalis x S. viminalis, Salix x smithiana clone S-218, Populus maximowiczii x Populus nigra | Zn, Cd, Pb | Phytoextraction | Zarubova et al., 2015 |
| Populus deltoides Dvina, Populus x canadensis Orion, Pteris vittata (Chinese brake fern) | As, Al, Fe, Cu, Co, Cr, Pb | co-planting system phytoextraction | Ciurli et al., 2014 |
| Industrial hemp and white lupin rotation | Zn, Cd, Ni, Cu | Winter crop/summer crop, phytoextraction | Fumagalli et al., 2014 |
| Rapeseed and rice rotation | Cd |  | Yu Lingling et al., 2014 |
| Sunflower |  | Secondary metal accumulator, phytoextraction | Kolbas et al., 2011 |
| Sunflower |  | Secondary metal accumulator, phytoextraction | Kidd et al., 2015 |
| maize (Zea mays L.), sunflower (Helianthus annuus L.), willow (Salix x smithiana Willd.), and poplar (Popuus nigra L. x P. maximowiezii) | Ni, Pb, Cd, Cr | phytoextraction | Kacalkova et al., 2014 |
| Maize, Vetiver | Pb | Phytoextraction aided by citric acid | Freitas et al., 2013 |
| Pelargonium | Pb, Cd, Zn, Cu, As | Phytoextraction | Shahid et al., 2012 |
| Nicotiana tabacum | Zn, Cd, Cu | Secondary metal accumulators, phytoextraction | Kidd et al., 2015; Gonsalvesh et al., 2016 |
| Brassica juncea | Cd | phytoextraction | Parisien et al., 2015 |
| Festuca arvernensis | Zn, Cd | Phytostabilisation | Frérot et al., 2006 |
| Miscanthus | Zn, Pb, Cd, etc. | Phytostabilisation | Nsanganwimana et al., 2014a, 2015, 2016; Kidd et al., 2015 |
| Arundo donax | Zn, Pb, Cd, etc. | Phytostabilisation | Nsanganwimana et al., 2014b |
| Holcus lanatus L. | Cd, Pb, Zn, As |  | Friesl-Hanl et al., 2009 |
| Salix spp., Populus nigra L., Agrostis capillaris L. | Cu | (aided) phytostabilisation | Touceda-Gonzales et al., 2017a |
| Andropogon schirensis, Eragrostis racemosa, Loudetia simplex | Cu | phytostabilisation | Boisson et al., 2016 |

Supplement Tab.2 Phytomanagement experiments at field scale established before 2015, including phytoextraction, phytostabilisation and in situ stabilization/phytoexclusion options, were reviewed by Vangronsveld et al., (2009), Mench et al., (2010) and Kidd et al., (2015). Precautions for use are needed with invasive plant species (Che-Castaldo and Inouye, 2015; Losfeld et al., 2015). White lupin as winter crop in sequence with a metal-accumulator summer crop such as industrial hemp can improve the recovery of soil quality during the phytoextraction period, i.e. green manure avoiding the application of chemical amendments, increase in soil bacteria and metal bioavailability (Fumagalli et al., 2014). Planting oilseed rape increased the uptake of Cd by the successive rice crop compared with a previous fallow treatment (Yu Lingling et al., 2014). The beneficial effect of compost incorporation into Cu-contaminated soils, alone and in combination with other amendments, and crop rotation is generally marked on both plant biomass and microbial communities (Kolbas et al., 2011; Touceda-Gonzales et al., 2017a). Hyperaccumulators are currently developed for Ni phytomining and producing biosourced Cu-, Mn, and Co-ecocatalysts for the green chemistry (Losfeld et al., 2015). Biocatalysis, based on use of metal(loid) species originating from plant biomasses with high metal(loid) concentrations (unusual oxidation levels, new associated chemical species, and effects of synergy) is an emerging technique (Grison et al., 2015a). Intercropping with Fabaceae is relevant in case of mixed soil contamination. For example, liming with Sedum plumbizincicola intercropped with Medicago sativa enhanced soil PCB degradation by 25% and removal rates of Cd and Cu (Wu et al., 2012). Co-planting system, i.e. As-hyperaccumulator fern Pteris vittata and poplar clones, was suitable for phytoextracting As and metals at a contaminated dumping site, albeit the efficiency depended on irrigation, soil tillage and soil amendments for enhancing plant growth (Ciurli et al., 2014). Co-planting S. alfredii and Alocasia marorrhiza decreased the DTPA-extractable Zn, Cd, and Cu and benzo[a]pyrene in a sludge as compared with the unplanted sludge (Qiu et al., 2014). In a pot experiment, remediation of a spiked Cd and PAH co-contaminated soil by S. alfredii was enhanced by application of pig manure vermicompost (Wang et al., 2012).

Supplementary Table 3. Selected indicators and measurements to assist smart intensification of crop production. The table also indicates clearly that intensification will have an influence on social and socioeconomic indicators. Both, driving forces and responses from stakeholders, policy makers and the local population are strong.

| Indicator | Trigger type D,P,S,I,R | Related soil measurements | Related meteorological measurements | Other related assessments |
| --- | --- | --- | --- | --- |
| Ecological indicators |  |  |  |  |
| Carbon cycling | D, R | Soil org matter, microbial biomass, microbial community composition | Soil temperature, precipitation |  |
| Soil fertility | S | Nutrient analyses, microbial activity, organic matter, rooting depth, pH etc | Soil temperature, precipitation |  |
| Soil resilience | S | Soil erosion, soil compaction, organic carbon, pH, N and other nutrients, | Soil temperature, precipitation | Nutrient contents, water retention |
| Biodiversity | P, R | Plant species, soil fauna, microbial biomass, microbial community composition; fungal/bacterial community biomass ratio, |  |  |
| Biogeophysical indicators |  |  |  |  |
| Carbon footprint | D, R | Soil GHG emissions, C/N-ratio, DOM | Temp., precipitation, solar radiation |  |
| Water quality | S | Concentration of N, P, pesticides etc in water | Rainfall frequency, surface runoff |  |
| Soil physical properties | S | Bulk density, texture, electrical conductivity, infiltration rate, soil aggregates | Soil temp., conductivity | Infiltration, oxygen levels, texture composition |
| Economic indicators |  |  |  |  |
| Yield quantity and quality | D, I | Crop yield, nutritive value and contamination | Temp., precipitation, solar radiation |  |
| Profitability | D, R |  |  | annual income from farm |
| Land value / ownership | D, R |  |  | farm real estate data, rent data |
| Health of population | D |  |  |  |
| Economic growth | D, R |  |  | market price of harvests |
| Social indicators |  |  |  |  |
| Status of infrastructure | D, R |  |  | public/private transport |
| Sustainable human activity | D, R |  |  |  |
| Job opportunities | D, I, R |  |  | unemployment rate |
| Local interest (locally important speciality crops) | D, I, R |  |  | tourism? |

References to the supplementary section:

Antonkiewicz, J., Kołodziej, B., and Bielińska, E.J., 2017. Phytoextraction of heavy metals from municipal sewage sludge by Rosa multiflora and Sida hermaphrodita. Int. J. Phytorem., 19(4), 309-318.

Bert, V., Allemon, J., Sajet, P., Dieu, S., Papin, A., Collet, S., Raventos, C., 2017. Torrefaction and pyrolysis of metal-enriched poplars from phytotechnologies: Effect of temperature and biomass chlorine content on metal distribution in end-products and valorization options. Biomass and Bioenergy, 96, 1-11.

Boisson, S., Le Stradic, S., Collignon, J., Séleck, M., Malaisse, F., Shutcha, M. N., ... Mahy, G., 2016. Potential of copper-tolerant grasses to implement phytostabilisation strategies on polluted soils in South DR Congo. Env.Sci Pollut. Res., 23(14), 13693-13705.

Ciurli, A., Lenzi, L., Alpi, A., Pardossi, A., 2014. Arsenic uptake and translocation by plants in pot and field experiments. Int. J. Phytorem., 16(7-8), 804-823.

EPA, 2002. Biosolids Applied to Land: Advancing Standards and Practices. National Academies Press, Washington DC, USA.

Escande, V., Petit, E., Garoux, L., Boulanger, C., Grison, C., 2015. Switchable alkene epoxidation/oxidative cleavage with H2O2/NaHCO3: Efficient heterogeneous catalysis derived from biosourced Eco-Mn. ACS Sust. Chem.Engin., 3(11), 2704-2715.

Freitas, E.V., Nascimento, C.W., Souza, A., Silva, F.B., 2013. Citric acid-assisted phytoextraction of lead: a field experiment. Chemosphere, 92(2), 213-217.

Friesl-Hanl, W., Platzer, K., Horak, O., Gerzabek, M. H., 2009. Immobilising of Cd, Pb, and Zn contaminated arable soils close to a former Pb/Zn smelter: a field study in Austria over 5 years. Environ. Geochem. Health, 31(5), 581-594.

Fumagalli, P., Comolli, R., Ferre, C., Ghiani, A., Gentili, R., Citterio, S., 2014. The rotation of white lupin (Lupinus albus L.) with metal-accumulating plant crops: A strategy to increase the benefits of soil phytoremediation. J Environ. Manag., 145, 35-42.

Gonsalvesh, L., Yperman, J., Carleer, R., Mench, M., Herzig, R., Vangronsveld, J., 2016. Valorisation of heavy metals enriched tobacco biomass through slow pyrolysis and steam activation. J. Chem. Tech. Biotech., 91(6), 1585-1595.

Kacálková, L., Tlustoš, P., Száková, J., 2014. Chromium, Nickel, Cadmium, and Lead Accumulation in Maize, Sunflower, Willow, and Poplar. Polish J. Env. Stud., 23(3).

Kacálková, L., Tlustoš, P., Száková, J., 2015. Phytoextraction of risk elements by willow and poplar trees. Int. J. Phytorem., 17(5), 414-421.

Keeling, S.M., Stewart, R.B., Anderson, C.W.N., Robinson, B.H., 2003. Nickel and cobalt phytoextraction by the hyperaccumulator Berkheya coddii: implications for polymetallic phytomining and phytoremediation. Int. J. Phytorem., 5(3), 235-244.

Kolbas, A., Mench, M., Herzig, R., Nehnevajova, E., and Bes, C. M., 2011. Copper phytoextraction in tandem with oilseed production using commercial cultivars and mutant lines of sunflower. Int. J. Phytorem., 13(sup1), 55-76.

Kubátová, P., Hejcman, M., Száková, J., Vondráčková, S., Tlustoš, P., 2016. Effects of sewage sludge application on biomass production and concentrations of Cd, Pb and Zn in shoots of Salix and Populus clones: Improvement of phytoremediation efficiency in contaminated soils. BioEnergy Res., 9(3), 809-819.

Losfeld, G., Mathieu, R., L’Huillier, L., Fogliani, B., Jaffré, T., Grison, C., 2015. Phytoextraction from mine spoils: insights from New Caledonia. Env. Sci. Poll. Res., 22(8), 5608-5619.

Losfeld, G., delaBlache, P.V., Escande, V., Grison, C., 2012. Zinc hyperaccumulating plants as rene-wable resources for the chlorination process of alcohols. Green Chem. Lett. Rev., 5(3), 451-456.

Mahieu, S., Soussou, S., CleyetMarel, J. C., Brunel, B., Mauré, L., Lefèbvre, C., Escarré, J., 2013. Local adaptation of metallicolous and non-metallicolous Anthyllis vulneraria populations: their utilization in soil restoration. Restoration Ecol., 21(5), 551-559.

Niazi, N.K., Singh, B., Van Zwieten, L., Kachenko, A.G., 2012. Phytoremediation of an arsenic-contaminated site using Pteris vittata L. and Pityrogramma calomelanos var. austroamericana: a long-term study. Env. Sci. Pollut. Res., 19(8), 3506-3515.

Nsanganwimana, F., Waterlot, C., Louvel, B., Pourrut, B., Douay, F., 2016. Metal, nutrient and biomass accumulation during the growing cycle of Miscanthus established on metal‐contaminated soils. J Plant Nutr. Soil Sci, 179(2), 257-269.

Nsanganwimana, F., Pourrut, B., Waterlot, C., Louvel, B., Bidar, G., Labidi, S., ...Douay, F., 2015. Metal accumulation and shoot yield of Miscanthus× giganteus growing in contaminated agricultural soils: Insights into agronomic practices. Agric. Ecosys. Env., 213, 61-71.

Nsanganwimana, F., Pourrut, B., Mench, M., Douay, F., 2014a. Suitability of Miscanthus species for managing inorganic and organic contaminated land and restoring ecosystem services. A review. J. Env. Manag., 143, 123-134.

Nsanganwimana, F., Marchand, L., Douay, F., Mench, M., 2014b. Arundo donax L., a candidate for phytomanaging water and soils contaminated by trace elements and producing plant-based feedstock. A review. Int. J. Phytorem., 16(10), 982-1017.

Parisien, M.A., Rutter, A., Zeeb, B.A., 2015. Feasibility of using phytoextraction to remediate a compost-based soil contaminated with cadmium. Int. J. Phytorem., 17(11), 1137-1143.

Qiu, J.R., Guo, X.F., Cai, Q.Y., Liu, W., Zhang, M.W., Wei, Z.B., Wu, Q.T., 2014. Phytotreatment of sewage sludge contaminated by heavy metals and PAHs by co-planting Sedum alfredii and Alocasia marorrhiza. Int. J. Phytorem., 16(1), 1-13.

Shahid, M., Arshad, M., Kaemmerer, M., Pinelli, E., Probst, A., Baque, D., ... Dumat, C., 2012. Long-term field metal extraction by Pelargonium: phytoextraction efficiency in relation to plant maturity. Int. J. Phytorem., 14(5), 493-505.

Tlustoš, P., Břendová, K., Száková, J., Najmanová, J., Koubová, K., 2016. The long-term variation of Cd and Zn hyperaccumulation by Noccaea spp and Arabidopsis halleri plants in both pot and field conditions. Int. J. Phytorem., 18(2), 110-115.

Vangronsveld, J., Herzig, R., Weyens, N., Boulet, J., Adriaensen, K., Ruttens, A., Thewys T., Vassilev, A., Meers, E., Nehnevajova, E., van der Lelie, D., Mench M. 2009. Phytoremediation of contaminated soils and groundwater: lessons from the field. Env. Sci. Pollut. Res., 16(7), 765-794.

Wang, K., Zhang, J., Zhu, Z., Huang, H., Li, T., He, Z., ... Alva, A., 2012. Pig manure vermicompost (PMVC) can improve phytoremediation of Cd and PAHs co-contaminated soil by Sedum alfredii. JSS, 12(7), 1089-1099.

Wu, L., Li, Z., Han, C., Liu, L., Teng, Y., Sun, X., ... Christie, P., 2012. Phytoremediation of soil contaminated with cadmium, copper and polychlorinated biphenyls. Int. J. Phytorem., 14(6), 570-584.

Yu, L., Zhu, J., Huang, Q., Su, D., Jiang, R., Li, H., 2014. Application of a rotation system to oilseed rape and rice fields in Cd-contaminated agricultural land to ensure food safety. Ecotox. Env. Safety, 108, 287-293.

Zárubová, P., Hejcman, M., Vondráčková, S., Mrnka, L., Száková, J., & Tlustoš, P., 2015. Distribution of P, K, Ca, Mg, Cd, Cu, Fe, Mn, Pb and Zn in wood and bark age classes of willows and poplars used for phytoextraction on soils contaminated by risk elements. Env. Sci. Pollut. Res., 22(23), 18801-18813.