Supplement 3A In vitro methods primary human hepatocytes and HepG2

SOP: Cryopreserved primary human hepatocyte cultivation in collagen monolayer system	1

SOP: Cryopreserved primary human hepatocyte cytotoxicity testing with CellTiter-Blue viability assay	6

SOP: Isolation of primary human hepatocytes	11

SOP: Primary human hepatocyte cultivation in collagen sandwich system	18

SOP: Primary human hepatocytes rna harvesting and isolation with TRIzol/QIAzol	24

SOP: cDNA Synthesis	27

SOP: Quantitative real time PCR	29

SOP: HepG2 cultivation and passaging	30

SOP: HepG2 cells cytotoxicity testing with CellTiter-Blue viability assay	32

SOP: HepG2 cells collection of samples for gene expression analysis	37

SOP: HepG2 cells glutathion depletion assay	39

SOP: CRYOPRESERVED PRIMARY HUMAN HEPATOCYTE CULTIVATION IN COLLAGEN MONOLAYER SYSTEM (Version 20170824)

# Equipment

- Laminar airflow cabinet

- Incubator (37°C, humidified atmosphere of air containing 5% CO2)

- Temperature-controlled bath (37°C)

- Bright field / phase contrast microscope

- Hemocytometer Neubauer chamber, 0.100 mm depth, 0.0025 mm2 (BLAUBRAND Wertheim, Germany)

- Pipet-aid, pipettes and micropipettes

- Polystyrene tubes (50 mL)

- 96-well plates

- Sterile pipette tips with cut tip (bigger hole)

- Ice box

# Substances

- Rat collagen lyophilized (Roche Diagnostic Mannheim, 10 mg, Cat. No. 11171179001)

- 0.2% solution of acetic acid (v/v in dH20, sterile filtered)

- William’s E cultivation medium with additives (Table 3.1)

- 10x Dulbecco’s Modified Eagle Medium [DMEM] (BioConcept, Allschwil, Switzerland Cat. No. 1-25K03-I)

- 0.4 % Trypan blue solution (Sigma Aldrich Corp., St. Louis, MO, USA, product number T8154)

- Sterile phosphate-buffered saline (PBS)

# Medium

Primary human hepatocytes are cultivated in William's E Medium, PAN Biotech GmbH, Aidenbach, Germany (Catalog Number P04_29510). All media should be stored at 4°C but pre-warmed at 37°C prior to use.

## Cultivation medium

For normal cultivation of cells, the following additives are included

Table 1: Medium for cultivation of primary human hepatocytes

| Additives | Company | Catalog Number | Final concentration | For 500 mL Medium |
| --- | --- | --- | --- | --- |
| Penicillin/Streptomycin | PAN Biotech GmbH, Aidenbach, Germany | P06-07100 | 100 U/mL Pen 0.1 mg/mL Strep | 5 mL |
| Gentamicin | PAN Biotech GmbH, Aidenbach, Germany | P06-13001 | 10 µg/mL | 500 µL |
| Dexamethasone | Sigma-Aldrich Corp., St. Louis, MO, USA | D4902-25MG | 100 nM | stock in EtOH |
| Stable L-Glutamine | Sigma-Aldrich Corp., St. Louis, MO, USA | P04-82100 | 2 mM | 5 mL |
| Insulin supplement (ITS) | Sigma-Aldrich Corp., St. Louis, MO, USA | 3146 | 2 ng/mL | 5 µL |

## Full medium

Cultivation medium (3.1) supplied with 10% Sera Plus (PAN Biotech GmbH, Aidenbach, Germany, Cat. No. 3701-P103009)

This medium is used for plating cells, the addition of Sera Plus is required for the attachment of cells to the collagen coated plate.

# Cultivation of cryopreserved primary hepatocytes in collagen monolayer-coated 96-well plates

Primary hepatocytes are seeded in collagen-coated 96-well plates, 50,000 cells in 0.2 mL of full medium (3.2) per well. Before the cells are brought into culture, several preparation steps need to be performed. Lyophilized collagen needs to be dissolved for at least 3 h and plates need to be coated with a thin film of collagen. After washing the plates, the cells can be seeded. All of the following steps should be performed under sterile conditions. The collagen solution should be constantly kept on ice to prevent gelation process.

## Preparation of the collagen monolayer

In a first step lyophilized collagen needs to be dissolved. The collagen is delivered in bottles with 10 mg of lyophilized rat tail collagen, which needs to be dissolved in 40 mL sterile filtered 0.2% acetic acid per bottle. This results in a final collagen concentration of 250 µg/mL. The collagen is dissolved for at least 3 h at 4°C until no collagen clumps are visible. Ideally, the collagen is dissolved overnight. Dissolved collagen is rapidly stored at 4°C and kept for maximum one month. Do not re-use afterwards.

When the collagen is properly dissolved, plates are coated with a thin layer of collagen by adding 100 µL per well in a 96-well format. Multichannel pipets are used to transfer the collagen solution from well to well, leaving a thin film of collagen in each well. Coated plates need to dry, either overnight with closed lids or for at least 1.5-2 h with open lid under the sterile hood.

Afterwards three washing steps with sterile PBS are performed. Finally, 200 µL of cultivation medium including 10% FBS are added to each well. Store the plates at 37°C until cells are seeded.

Note: A medium change is carried out by turning the plate upside down on a piece of tissue paper. Aspirating the liquid with a cell culture pump is not recommendable because cells are removed easily from the plate.

## Quality check and handling of cryopreserved primary human hepatocytes

Cryoplateabale primary human hepatocytes are purchased from BioIVT (Westbury, New York, US), which freshly isolate and cryopreserve the hepatocytes on the same day. Cells are delivered in cryoshippers and stored in liquid nitrogen until usage.

Before ordering larger batches, single vials with hepatocytes from individual donors are tested regarding their quality and their ability to attach. The quality and viability of the cells differs from donor to donor, therefore the cells from individual donors are tested prior to ordering larger amounts. Donor specific properties such as the fattiness of the tissue strongly influence the ability of the cells to attach to the collagen monolayer.

Avoid cells from donors with a strong alcohol consumption. Do not use cells with a viability less than 60%.

Figure 1: Examples for cultivated cryopreserved primary human hepatocytes which (A) fulfil the quality criteria and (B) do not fulfill the quality criteria.

Each vial of cryopreserved primary human hepatocytes contains approximately 5 million viable hepatocytes, guaranteed by the company. For plating the cells are thawed in a water bath for 1 – 2 min. The cell suspension should not fully warm up, but defrost. For each vial of 5 million cells, five mL of warm full medium is given into a 50 mL Falcon tube. Afterwards the defrosted cell suspension is transferred into the tube. Do not harshly pipette up and down, but transfer the cells by pouring or pipetting with a cut tip (bigger hole). The cell suspension is kept at room temperature until plating.

## Trypan blue stain for viability check and counting

- Prepare a 1:10 dilution of your cell suspension by gently mixing 350 µL medium with 100 µL 0.4% Trypan blue solution and 50 µL cell suspension. Gently pipette up and down but avoid harsh pipetting.

- Quickly fill the 1:10 diluted Trypan blue cell suspension into a hemocytometer and count the amount of viable and dead (blue) cells. Cells in Trypan blue solution rapidly loose viability, therefore the counting should be performed directly after preparation of the 1:10 dilution mixture. The dilution process and the counting itself should not exceed more than 5 min duration in total.

- Count the cells under a bright field microscope by using a hemocytometer. Fill 10 µL of the diluted Trypan blue cell suspension into both chambers of the hemocytometer. Count cells in all 4 quartiles in both chambers (each quartile consists of 4x4 smaller squares, see Figure 2, marked in yellow) and note viable as well as dead blue cells. Calculate the average of viable cells per quartile and determine the cell viability as given below. Do not use hepatocytes with viability less than 60%. In case the viability is higher than 60% you can continue and plate the cells.

Figure 2: Example for counting cells with a Neubauer chamber (0.100 mm depth)

- Determine the viability of the cells as follows:

Viability= average living cells per quartileaverage total cells per quartile x  100%

The viability of the example shown above is 78.625 / 83.125 x 100% = 94.6%.

- Calculate the amount of vital cells per mL as follows:

Cells per mL =average living cells per quartile x dilution factor x 10,000

Using a dilution factor of 10, the amount of cells per mL in the above mentioned example corresponds to 78.625 x 10 x 10,000 = 7.8625 x 106 cells.

- Calculate the amount of cells (50,000) that is required for each well. Calculate for 100% viability:

50,000 cells=1000amount of cells per mL x 50,000 viability x 100

## Seeding cryopreserved primary human hepatocytes

- 50,000 cells per well are transferred to the collagen monolayer-coated 96-well plates with 200 µL full medium per well. Use cut tips and a multichannel pipette to transfer the required amount of freshly isolated hepatocyte cell suspension to each well. Make sure that the cell suspension is well mixed, therefore gently pipette the cell suspension up and down every 30-60 seconds.

- When the plate is completed, shake the plate gently side to side (in directions north-south; west-east) until cells are evenly distributed.

- Transfer plates into the incubator for at least 3 h of attachment. During the first 20 min of attachment, shake the cells as described every 5 min.

- After at least 3 h (maximum 5 h) of attachment, gently remove the medium by turning the plate onto tissue paper.

- Carefully add 100 µL of PBS to each well. Wash each well 3 times with PBS.

- Finally add 200 µL of serum-free cultivation medium (3.1) to the cells and keep them at 37°C. Start with treatments in the morning (between 8 and 12 o’clock) the day after plating.

SOP: CRYOPRESERVED PRIMARY HUMAN HEPATOCYTE CYTOTOXICITY TESTING WITH CELL-TITER-BLUE VIABILITY ASSAY (Version 20170824)

Cytotoxicity tests (CellTiter-Blue viability assays) are performed in three independent experiments, meaning three biological replicates with cells from different donors. Cells are cultivated in collagen coated 96-well plates, seeding 50,000 cells per well. For detailed description for hepatocyte plating and cultivation see SOP Hepatocyte cultivation in collagen monolayer system.

# Equipment

- Laminar airflow cabinet

- Incubator (water jacketed, 37°C, humidified atmosphere of air containing 5% CO2)

- Temperature-controlled bath (37°C)

- Pipet-aid, pipettes and micropipettes

- Polystyrene tubes (50 mL)

- Conventional cell culture microplates 96-well and black 96-well plates (Greiner bio-one, Frickenhausen, Germany, REF 655986)

- Infinite M200 Pro Plate reader (Tecan)

# Substances

- Cell Titer Blue Reagent from Promega (Cat. No. G8081; stored in aliquots at -20°C, thawed in the water bath before usage)

- William’s E cultivation medium with additives (see SOP Hepatocyte cultivation in collagen monolayer system)

- Test compounds

- PBS (Phosphate Buffered Saline)

# Time schedule and general overview

Cryopreserved primary human hepatocytes are seeded in conventional 96-well plates, 50,000 cells per well, as described in SOP: Cryopreserved primary human hepatocyte cultivation in collagen monolayer system (Page 1). Compound treatment starts 16-20 h after plating cells. Five concentrations per compound plus solvent control are tested with four technical replicates per condition on each 96-well plate. Cells are exposed for 48 h to each test compound. Afterwards, each well is briefly considered under the microscope and a viability score is estimated (10 = all cells vital, 1 = all cells dead). Finally, the Cell Titer Blue viability assay is performed. Cell Titer Blue reagent is added to the cells for 2-4 h. After incubation at 37°C, the supernatant of each well is transferred to a 96-well plate and the fluorescence intensity is read out at 594 nm using the Tecan Infinite M200 Pro plate reader (i-control software (version 1.7.1.12)).

Figure 1: Time schedule and general overview for cytotoxicity tests in cultivated primary human hepatocytes

Day -1: Cells are thawed and brought into culture: 50,000 cells per well are plated on collagen monolayer coated 96-well plates.

Day 0: Start of compound treatment for 48 h. Each compound is tested in five different concentrations plus vehicle controls, 4 wells per condition.

Day 1: Incubation. Cell plates are left in the incubator.

Day 2: Check the morphology and evaluate on a scale from 1 – 10. (1 = all cells dead, 10 = all cells vital).

After 48 h of incubation: wash cells 3x with PBS before adding the Cell Titer Blue reagent in fresh medium. Cells will metabolize the CTB reagent between 2 and 4 h, which will result in a color shift. Afterwards the medium supernatant is transferred to black 96-well plates and fluorescence values are determined with a Tecan plate reader.

# Compound handling

- Compounds are stored according to the manufacturer’s instructions (4°C, room temperature or -20°C)

- Preferable solvent is DMSO, which is stored in a lightproof, air-tight bottle at room temperature.

- Stock solutions in DMSO are stored at -20°C.

- Final DMSO content for cellular exposure is 0.1%. However, in exceptional cases we increase the DMSO content up to 0.5% for the highest tested concentrations. We have tested to what extent different DMSO concentrations influence our assays. Up to 0.5% DMSO the read out of the assay does not change compared to lower DMSO concentrations.

- For all compounds a stock solution is prepared for each concentration. The highest concentrated stock solution serves as starting point to prepare lower concentrated stock solutions. Therefore an appropriate amount of the higher concentrated stock solution is added to DMSO in order to prepare the next lower concentrated stock solution.

- Some medium soluble compounds that were included from previous studies were dissolved in medium directly. In these cases, the test compounds were weighed and dissolved in medium directly, serving as starting point for a serial dilution to prepare lower concentrated solutions. An appropriate amount of the higher concentration was transferred to a tube with corresponding amounts of medium to prepare the next lower concentration.

# Treatment of the cells

Compound exposure starts between 16 and 20 h after plating the cells. Each compound is tested in 5 different concentrations (C1 (lowest) to C5 (highest conc.)) plus vehicle controls, 4 wells per condition. Therefore 24 wells have to be included for 1 biological replicate of a test compound.

All concentrations are tested on the same plate. A typical plate set up is shown in Figure 2 below. In cases where a DMSO concentration of 0.5% is required to dissolve the compound, an additional vehicle control with 0.5% DMSO is included.

Chose a dilution factor which is the same between two serial concentrations.

Figure 2: A typical plate set up (96-well format) for the Cell Titer Blue cytotoxicity testing in primary human hepatocytes.

Each well of the 96-well plate requires 200 µL of medium with the appropriate compound concentration or solvent control. The solutions are prepared by pipetting an appropriate amount of medium and adding the corresponding stock solution in DMSO. Mix well by vortexing. In exceptional cases where a compound is dissolved in medium directly, follow this instruction: Weigh the amount of the compound for the highest tested concentration (C5) and dissolve it in medium. Mix well by vortexing and store in the water bath for approximately 10 min. Sterile filter the solution and start a dilution series:  Prepare the next lower concentration C4 by mixing fresh cultivation medium with an appropriate amount of the highest concentration (C5). Continue the dilution series with the remaining concentrations, always by mixing medium with the appropriate volume of the next higher concentration. Therefore shake or vortex the next higher concentration before preparing the corresponding lower concentrated solution. When all dilution series are prepared, pre-warm the solutions in the water bath (37°C).

When all solutions are prepared, store them in the water bath (37°C) for approximately 10-15 minutes. Collect your plates from the incubator and discard the medium by turning the plate onto a tissue paper.

Add 200 µL of the appropriate solution to each well, starting with the solvent control and continue with increasing concentrations. Shake or vortex each concentration directly before adding it to the cells. This step should not take longer than 3 min.

Place the cells back into the incubator and incubate for 48 h

# Morphology Check

After 46-47 h of incubation, the cell morphology for each well is evaluated under the microscope. A viability score is assigned to each well with a scale from 1 - 10, matching a 10 for total viable and healthy cells and a 1 for cells which are dead. After the morphology check place the cells back into the incubator until the 48 h of exposure are complete.

# Cell Titer Blue Assay

The aim of the assay is to determine the test compound concentration that causes 20% loss of viability. Therefore the Cell Titer Blue (CTB) cytotoxicity assay from Promega is applied, which is based on mitochondrial enzyme activity and which analyzes the reduction rate of the dark blue indicator dye resazurin to the pink and highly fluorescent dye resorufin. While resazurin is only slightly fluorescent, resorufin is highly fluorescent and can be quantified at its emission maximum at a wavelength of 584 nm. Vital cells exhibit a strong metabolization capacity for resazurin, and the fluorescence intensity in the medium supernatant is increasing. In contrast, non-viable cells show a decreased resazurin metabolization capacity, resulting in lower fluorescence intensity or, in case of dead cells the dark blue resazurin is not even metabolized.

Before the 48 h of exposure are complete, CTB-medium is prepared for all plates as follows: Each well requires a total amount of 100 µL fresh CTB-medium. 100 µL CTB-medium is composed of 80 µL normal cultivation medium and 20 µL CTB reagent. Prepare a master mix for x+10 wells, you will need (x+10) * 0.08 mL medium and (x+10) * 0.02 mL CTB reagent. CTB reagent is light sensitive, therefore the light in the laminar airflow cabinet should be turned off while working with CTB or CTB-medium.

- After 48 h of compound exposure, the medium is carefully removed by turning the plate upside down on a piece of tissue paper.

- Cells are washed 3x with warm, sterile PBS.

- Subsequently 100 µL of CTB-medium is added to each well (light turned off!).

- Incubate at 37°C for 2-4 h. The exact incubation period is highly dependent on the individual donor. During the incubation time the dark blue indicator dye resazurin is metabolized to the pink and highly fluorescent dye resorufin. The color of the control medium will shift from blue (resazurin) to pink (resorufin) whereas the compound-exposed wells should ideally exhibit a color gradation from pink (control, low concentrations) to blue (high concentrations). Stop the metabolization and continue with step 5 as soon as a color gradient is visible by eye.

- The read out is performed in black 96-well plates (Greiner bio-one, Frickenhausen, Germany, REF 655986), therefore the supernatant medium of each well needs to be transferred to black 96-well plates. Work in the dark. Collect each plate individually from the incubator and place the cells on the normal work space area (unsterile working environment). Include at least 1 well of fresh CTB-medium that was not in contact with cells. This value is important for the background fluorescence.

- When all supernatants are transferred to the 96-well plate, the plate can be either read out immediately (see step 7) or kept aside in the dark for 20-30 min (until all remaining 96-well plates for the other compounds are complete).

- Use the Tecan Infinite M200 Pro plate reader (software i-control, version 1.7.1.12): Excitation wavelength 540 nm, emission wavelength 594 nm. The output is a fluorescence value for each well of the 96 well plate.

SOP: ISOLATION OF PRIMARY HUMAN HEPATOCYTES (Version 20160330)

# Buffers for perfusion

Table 1.1.: Composition of Perfusion Solution 1

| Ingredients for Perfusion Solution 1 | Concentration |
| --- | --- |
| NaCl | 142 mM |
| KCl | 6.7 mM |
| Hepes | 100 mM |
| N-Acetyl-L-cysteine | 5 mM |
| EGTA (ethylene glycol-bis(β-aminoethyl ether)-N,N,N′,N′-tetraacetic acid) | 2.4 M |
| Set pH to 7.5; perform sterile filtration; store at 4 °C |  |

Table 1.2.: Ingredients for Perfusion Solution 2

|  | Ingredients for Perfusion Solution 2 | Concentration |
| --- | --- | --- |
| Solution A | NaCl | 67 mM |
|  | KCl | 6.7 mM |
|  | Hepes | 100 mM |
|  | Albumin | 0.5% (w/v) |
| Solution B | CaCl2 | 4.8 mM |

Table 1.3: Composition of Perfusion Solution 2

| Preparation of Perfusion Solution 2 |  |
| --- | --- |
| Solution A | 1.3 L |
| Solution B | 150 mL |
| Fill up with distilled water | 1.5 L in total |
| Set pH to 7.5 |  |
| Perform sterile filtration |  |
| Store at 4°C |  |

Tab. 1.4.: Composition of the Stop solution

| Stop Solution Ingredients | Amount |
| --- | --- |
| FCS (Fetal Calf Serum) | 100 mL |
| Sterile PBS (Phosphate Buffered Saline) | 500 mL |
| Store solution at 4°C |  |

# Laboratory equipment and preliminary preparations

Note: In general, the isolation procedure has to be performed under sterile conditions. Therefore, it is advised to place the whole perfusion system set-up under the sterile workbench. In order to ensure a smooth and successful performance of the isolation procedure, equipment and all necessary solutions have to be prepared sterile and should be ready to use.

- The following material should be placed on an unwrapped surgical drape sheet: forceps; scalpel; gauze; glass petri dish; buttoned cannula with up to 6 channels (Fig. 1a)

- Set the water bath temperature to 39°C. Keep the temperature at the outflow between 36 and 38°C in order to maintain an optimal collagenase activity

- Prepare a beaker with ice and several 50 mL tubes. The number of tubes depends on the size of the tissue to be isolated. Approximately 10 tubes for a 40 g piece of perfused liver tissue are advisable.

- For the filtration of the cell suspension subsequent to the isolation procedure, a sterile plastic funnel coated with aseptic gauze is used (Fig. 1c).

Figure 1: Liver cell isolation set-up. (a) Ready-to-use sterile work area with sterile drape sheet, glass and plastic dishes, scalpel, forceps, cell scraper, gauze, and cannula for perfusion. (b) Perfusion system with water bath, thermostat, peristaltic pump and Buchner funnel. (c) Plastic funnel with aseptic gauze and 50 mL tubes for the filtration of the cell suspension after the isolation procedure.

Further preparations:

- A bottle with Perfusion Solution 1 is placed in the water bath and fixed with a lead ring to prevent floating

- Secure a Buchner funnel with a tripod and place an empty sterile 500 mL bottle underneath for waste collection.

- Place both ends of a sterile silicone tube in the bottle containing perfusion solution I and let the solution circulate until the hose is completely filled (Fig. 1b,). Avoid air bubbles, because they might be harmful to the tissue.

# General information for the isolation of primary human hepatocytes

The isolation of primary human hepatocytes was performed for the first time more than 40 years ago (Bojar et al. 1976; Strom et al. 1982; Guguen-Guillouzo et al. 1982; Reese and Byard 1981). The introduction of the two-step isolation procedure using collagenase by Seglen et al. (1976) signified an important progress in primary liver cell isolation. This procedure has been the basis of many isolation protocols that have been published in the past 40 years (Dorko et al. 1994; Strom et al. 1982; Strom et al. 2005; Knobeloch et al. 2011; Gerlach et al. 1994).

The ideal tissue for liver cell isolation weighs between 30 to 100 g and is surrounded on three sides by tissue capsule from a patient who is less than 50 years old with a single isolated pathological finding (e.g. single metastasis or liver tumor such as CCC) or trauma (Fig. 2a). Usually, the smaller the resection area, the better is the performance of the perfusion (Fig. 2b).

Figure 2: (a) Resected liver piece on a sterile work area in the surgical suite showing a colorectal liver metastasis (see arrow) after right-lateral hemihepatectomy. (b) Preparation of the piece of liver tissue that is used for hepatocyte isolation by using an amputation knife.

# Isolation procedure

Note: The resected liver tissue should be transported as fast as possible into a sterile vessel containing cell culture medium or PBS on ice in order to minimize the time of warm ischemia. The isolation procedure starts immediately after the arrival at the laboratory.

- The tissue is removed from the transport container and placed on a sterile Petri dish followed by a	gentle wipe of the remaining blood from the tissue using an aseptic gauze.

In order to prepare the piece of liver for perfusion I, the buttoned cannulae are placed into several vessels of the resection site and fixed with tissue glue (B.Braun, Melsungen, Germany). By using the biological architecture of vessels, the whole piece of liver can be perfused with 3 to 8 cannulae depending on the size of the tissue. Using too many cannulae can restrain the outflow of the perfusion solution or lead to a long recirculation of the perfusion solution into the tissue.

All cannulae need to be checked for blockages by pumping perfusion solution through the system. Afterwards cannulae are placed into the largest vessels and fixed by using tissue glue (Fig. 3a). Smaller vessels and bile ducts should be closed in the same way. Control the perfusion pressure and adjust the pump speed up to the point that a homogenous perfusion is guaranteed.

During the first perfusion step, the remaining blood is removed and the liver tissue is prepared for the collagenase digestion. EGTA is added to prevent the coagulation of the residual blood and to deplete cellular Ca2+. Furthermore, N-Acetyl-L-Cysteine can be used to prevent oxidative damage in the cells during the perfusion process (Stephenne et al. 2007).

- Transfer the prepared tissue into the Buchner funnel and start perfusion step I. The set-up for this perfusion is depicted in Fig. 3 b. At least 300 mL of perfusion solution I are required in order to remove residual blood and to warm up the tissue. The end of this perfusion step is indicated by a light-red color of the outflowing perfusion solution I.

Note: In order to avoid a loss of perfusion pressure, any vessel leakage should be immediately stopped by using additional tissue glue.

- During the first perfusion step it is recommendable to prepare the buffer for perfusion step II by adding the necessary amount of Collagenase P for the digestion of the liver tissue to 100 mL Perfusion Solution 2 (pre-warmed at 37°C). The solution is sterile filtered into a glass bottle of at least 150 mL and kept warmed at 37°C in order to avoid any loss of activity.

Figure 3: (a) A buttoned cannula is placed in a blood vessel and fixed with tissue glue. (b) The first perfusion step is carried out in order to remove residual blood and to warm up the tissue. (c) Recirculation of Collagenase P-containing perfusion solution for digestion of the liver tissue during perfusion step II. (d) The digested liver tissue is gently tweezed and cut into two halves with a scalpel, before the cells are released into the surrounding stop solution.

For the collagenase digestion, the perfusion set-up has to be rearranged in order to enable the recirculation of the perfusion solution II, which reduces the necessary amount of Collagenase P and hence reduces the running cost (Fig. 3c):

- The silicone tube is removed from the cannula tubing and emptied. Afterwards the end of the tube is transferred from perfusion solution I into the bottle containing Perfusion Solution 2.

- Remove the bottle for the collection of waste and the bottle containing perfusion solution I from the water bath and place the bottle with the collagenase buffer (perfusion solution II) underneath the Buchner funnel. Then, re-attach the silicone tube to the buttoned cannula and start the perfusion. Check all adhesive joints for leakages and, if necessary, re-fix the cannula with tissue glue.

The termination of perfusion step II is indicated by irreversible deformation of the tissue upon slight pressure. This state should be reached after approximately 10 to 20 min.

- Following the digestion procedure, the cannulae have to be pulled out quickly. The tissue is placed into the Petri dish and stop solution is immediately poured over the tissue for enzyme inactivation. This inactivation prevents the over-digestion of the tissue.

- In order to release the hepatocytes, a scalpel is used to cut the piece of perfused liver into two halves without cutting through the capsule (Fig. 3d). Tweeze the tissue gently and release the cells from the tissue into the surrounding stop solution.

- Collect the cell suspension with a serological pipette and pass the suspension through the prepared funnel with gauze into ice-cooled 50 mL tubes in order to eliminate tissue debris.

- Pellet cells by centrifugation with a speed ranging between 50-100 x g at 4°C for 5 min. With cells from steatotic livers a higher centrifugation speed (up to 100 x g) is recommended, since these cells often contain intracellular fat that will hinder the sedimentation of the cells at 50 x g.

The supernatant is aspirated and discarded and the cell pellets can be re-suspended in hepatocyte culture medium or PBS and placed on ice.

A complete perfusion of liver tissue can nicely be seen in Fig. 4. Only the left side of the liver tissue has been well perfused and shows the typical morphology of digested liver tissue after harvesting the hepatocytes. The perfusion of the right side, however, has not been sufficiently perfused and therefore, almost no cells could be isolated from this part of the tissue.

Figure 4: Digested liver tissue: The left side of the tissue shows the typical morphology of digested tissue matrix after harvesting the hepatocytes. The right side, however, has not been sufficiently perfused and therefore the tissue remains intact.

Cell yield and viability:

In order to assess yield and viability, Trypan blue exclusion was performed. Vital and total amount of cells were counted using a Neubauer hemocytometer. The number of total cells per mL and the cell viability is determined according to the following formulas:

Total cells per mL=counted cells ×104 ×dilution factor

cell viablity (%)= vital cells  ×100total cells

With this protocol it is possible to obtain a cell viability of over 90%. However, the viability and quality of hepatocytes can be impaired if there is prolonged ischemia or if the donor had a steatotic or fibrotic liver. If the cell suspension contains less than 70% of viable cells, a separation of the dead cells from the viable cells by density gradient centrifugation with Percoll is highly recommended.

# Primary human hepatocyte purification with Percoll®

The quality of the isolated hepatocytes strongly depends on many factors including initial liver damage (normal, steatotic, cholestatic or fibrotic tissue), the conditions caused by the liver disease itself, and the pretreatment of the liver by drugs as well as the warm ischemia time upon surgery. In conclusion, not only the cell yield but also the cell viability varies strongly between different isolations.

The density gradient centrifugation with Percoll can be performed and is strongly recommended if the initial viability is below 70%. The best separation has been observed with 25% Percoll solution (15 mL PBS + 5 mL Percoll) and no more than 50 x 106 viable cells in a total volume of 4-5 mL.

Depending on the required amount of cells, several 50 mL Falcon tubes are filled with 25% Percoll solution and carefully overlaid with the cell suspension (Fig. 5a). The cell suspension is centrifuged in the Percoll density gradient at 1200-1300 x g at 4°C for 15-20 min. In order to improve the separation, minimum acceleration and no break are recommendable. After centrifugation dead cells and cell debris are located at the interphase, while viable cells are pelleted (Fig. 5 b). Both layers are aspirated and the cell pellet is re-suspended in PBS. The Falcon is filled with PBS to the 50 mL mark and centrifuged again at 50-100 x g at 4°C for 5 min. The supernatant is discarded and the pellets are re-suspended in an appropriate media for the culture of hepatocytes. Yield and viability of hepatocytes will be assessed again using Trypan blue exclusion as described above.

At this point the cells can be stored on ice for several hours prior to seeding or shipping.

Figure 5: Separation of viable and dead hepatocytes by density gradient centrifugation. (a) A 25-% Percoll solution is carefully overlaid with the suspension containing a maximum of 50 million viable cells. (b) After the centrifugation, the dead cells and the cell debris are located at the interphase, while viable cells are pelleted.

References:

Bojar H, Basler M, Fuchs F, Dreyfurst R, Staib W, Broelsch C (1976) Preparation of parenchymal and non-parenchymal cells from adult human liver--morphological and biochemical characteristics. J Clin Chem Clin Biochem 14 (11):527-532.

Dorko K, Freeswick PD, Bartoli F, Cicalese L, Bardsley BA, Tzakis A, Nussler AK (1994) A new technique for isolating and culturing human hepatocytes from whole or split livers not used for transplantation. Cell Transplant 3 (5):387-395.

Gerlach JC, Brombacher J, Kloppel K, Schnoy N, Neuhaus P (1994) Comparison of four methods for mass hepatocyte isolation from pig and human livers. Transplantation 57 (9):1318-1322.

Guguen-Guillouzo C, Campion JP, Brissot P, Glaise D, Launois B, Bourel M, Guillouzo A (1982) High yield preparation of isolated human adult hepatocytes by enzymatic perfusion of the liver. Cell Biol Int Rep 6 (6):625-628.

Knobeloch D, Ehnert S, Schyschka L, Buchler P, Schoenberg M, Kleeff J, Thasler W, Nussler NC, Godoy P, Hengstler JG, Nussler AK (2011) Human hepatocytes: Isolation, culture and quality procedures. In: Mitry RR, Hughes RD (eds) Human Cell Culture Protocols, vol 806. Springer Science+Business Media, pp 99-120

Reese JA, Byard JL (1981) Isolation and culture of adult hepatocytes from liver biopsies. In Vitro 17 (11):935-940.

Stephenne X, Vosters O, Najimi M, Beuneu C, Dung KN, Wijns W, Goldman M, Sokal EM (2007) Tissue factor-dependent procoagulant activity of isolated human hepatocytes: relevance to liver cell transplantation. Liver Transpl 13 (4):599-606.

Seglen PO (1976) Preparation of isolated rat liver cells. Methods Cell Biol 13:29-83.

Strom SC, Dorko K, Thompson MT, Pisarov LA, Nussler AK (2005) Large scale isolation and culture of human hepatocytes. Ilots de Langerhans et hepatocytes:195-205.

Strom SC, Jirtle RL, Jones RS, Novicki DL, Rosenberg MR, Novotny A, Irons G, McLain JR, Michalopoulos G (1982) Isolation, culture, and transplantation of human hepatocytes. J Natl Cancer Inst 68 (5):771-778.

SOP: PRIMARY HUMAN HEPATOCYTE CULTIVATION IN COLLAGEN SANDWICH SYSTEM (Version 20170824)

# Equipment

- Laminar airflow cabinet

- Incubator (water jacketed, 37°C, humidified atmosphere of air containing 5% CO2)

- Temperature-controlled bath (37°C)

- Bright field / phase contrast microscope

- Hemocytometer Neubauer chamber, 0.100 mm depth, 0.0025 mm2 (BLAUBRAND Wertheim, Germany)

- Pipet-aid, pipettes and micropipettes

- Polystyrene tubes (50 mL)

- 6-well plates

- Sterile pipette tips with cut tip (bigger hole)

- Cell scraper

- Ice box

# Substances

- Rat collagen lyophilized (Roche Diagnostic Mannheim, 10 mg, Cat. No. 11171179001)

- 0.2% solution of acetic acid (v/v in dH20, sterile filtered)

- William’s E cultivation medium with additives (Table 3.1)

- 10x Dulbecco’s modified Eagle medium [DMEM] (BioConcept, Allschwil, Switzerland Cat. No. 1-25K03-I)

- 1 M solution of NaOH for pH neutralization of collagen solution (sterile filtered)

- 0.4 % Trypan blue solution (Sigma Aldrich Corp., St. Louis, MO, USA, product number T8154)

# Medium

Primary human hepatocytes are cultivated in William's E Medium, PAN Biotech GmbH, Aidenbach, Germany (Catalog Number P04_29510). All media should be stored at 4°C but pre-warmed at 37°C prior to use.

## Cultivation medium

For normal cultivation of cells, the following additives are included:

Table 1: Medium for cultivation of primary human hepatocytes

| Additives | Company | Catalog Number | Final concentration | For 500 ml Medium |
| --- | --- | --- | --- | --- |
| Penicillin/Streptomycin | PAN Biotech GmbH, Aidenbach, Germany | P06-07100 | 100 U/mL Pen 0,1 mg/mL Strep | 5 mL |
| Gentamicin | PAN Biotech GmbH, Aidenbach, Germany | P06-13001 | 10 µg/mL | 500 µL |
| Dexamethasone | Sigma-Aldrich Corp., St. Louis, MO, USA | D4902-25MG | 100 nM | stock in EtOH |
| stable L-Glutamine | Sigma-Aldrich Corp., St. Louis, MO, USA | P04-82100 | 2 mM | 5 mL |
| Insulin supplement (ITS) | Sigma-Aldrich Corp., St. Louis, MO, USA | 3146 | 2 ng/mL | 5 µL |

## Full medium

Cultivation medium (Table 1) supplied with 10% Sera Plus (PAN Biotech GmbH, Aidenbach, Germany, Cat. No. 3701-P103009)

This medium is used for plating cells, the addition of Sera Plus is required for the attachment of cells to the collagen sandwich coated plate.

## Washing medium

William's E Medium, PAN Biotech GmbH, Aidenbach, Germany (Catalog Number P04_29510) without any additives.

# Cultivation of primary hepatocytes in collagen sandwich cultivation system (6-well format)

Primary hepatocytes are seeded in conventional 6-well plates, 1 million cells in 2 mL of full medium (3.2) per well between two soft layers of collagen gel. Before the cells are brought into culture, several preparation steps need to be performed. Lyophilized collagen needs to be dissolved for at least 3 h and afterwards the first layer of collagen has to be prepared, before cells can be seeded. All of the following steps should be performed under sterile conditions. All collagen solution should be constantly kept on ice to prevent gelation process.

## Preparation of the collagen sandwich

For the gel preparation, lyophilized collagen needs to be dissolved. The collagen is delivered in bottles with 10 mg of lyophilized rat tail collagen, which needs to be dissolved in 9 mL sterile filtered 0.2% acetic acid per bottle. The collagen is dissolved for at least 3 h at 4°C until no collagen clumps are visible. Ideally, the collagen is dissolved overnight. Dissolved collagen is rapidly stored at 4°C and kept for maximum 3 days. Do not re-use afterwards.

For both layers of the collagen sandwich, each well in a 6-well plate format requires 700 µL of collagen solution in total. One bottle of dissolved collagen is sufficient for both collagen layers of 2.5 of conventional 6-well plates.

When the collagen is properly dissolved, the first layer of the sandwich can be prepared. Usually the plates with the first layer of collagen (steps 1-6) are prepared in the morning of the day of plating, but they can be prepared and stored at 37°C (incubator) up to 24 h before plating the cells.

- Calculate the amount of collagen required for the first layer. For each well of a 6-well plate 350 µL of dissolved collagen are required, minus 1/10 of the volume that is required to add 10x DMEM.

x=number of wells x 0.35mL of dissolved collagen

required mL of collagen solution=x-(x10)

To avoid loss of collagen volume during the pipetting, always prepare gel for a couple of wells more than actually required.

- Dissolved collagen, sterile 1M NaOH, 10x DMEM and a 50 mL tube are placed on ice in a laminar flow air cabinet.

- Preparation of the gelled collagen solution: The calculated volume of required collagen solution is filled into the 50 mL tube on ice and mixed 1:10 with 10x DMEM. Afterwards the DMEM-collagen mixture is drop wise neutralized with 1M NaOH until the color turns from yellow to pink. The final collagen concentration is now 1 mg/mL. This gelled collagen solution needs to be freshly prepared for each layer of collagen. The mixture can be kept on ice for max. 1 h, but should not be re-used afterwards.

- Add 350 µL of the pH-adjusted gelled collagen (1mg/mL) to each well of the 6-well plate format. Distribute it evenly by quick movements of a cell scraper and gently shake the plate up and down and side to side until the bottoms of all wells are fully covered with collagen gel. Avoid air bubbles. The collagen solution polymerizes rapidly, therefore the preparation of this first layer should not exceed 1 min for each plate.

- Transfer plates with collagen into 37°C incubator for 35 - 45 min of polymerization. Place the rest of the 1 mg/mL gelled collagen in the tube on the side, no longer on ice as a polymerisation control.

- After successful polymerization gently add 2 mL of pre-warmed Full Medium (3.2) to each well. The polymerization process should not exceed 1h until medium is added. Keep the plates at 37°C until the cells are seeded.

## Handling of primary human hepatocytes upon arrival and quality check

Primary human hepatocytes are isolated from liver sections of patients undergoing surgical liver resection. For isolation of primary human hepatocytes see SOP Isolation of primary human hepatocytes (Page 11). Routinely, our lab receives isolated primary human hepatocytes from cooperating clinics which perfuse surgically obtained liver tissue and deliver the cells in suspension latest one day after the isolation process. Freshly isolated hepatocytes in suspension are constantly kept on ice until plating, therefore the cells are shipped on ice. The time between hepatocyte isolation and seeding should not exceed 24 h.

The following donor information is documented: age, gender, diagnosis, anamnesis (medical history), medication

Upon arrival of primary human hepatocytes, the following steps are performed:

- The vial with cells is carefully inverted to re-suspend the cells. Afterwards the cell suspension is transferred to a 50 mL tube on ice. Do not harshly pipette up and down. The vial is carefully rinsed with medium to transfer all cells to the 50 mL tube and the suspension is replenished to a total volume of 10 mL.

- Centrifugation at 4°C, 50xg for 5 min to pellet the cells. Afterwards the supernatant is aspirated and carefully 10 mL of normal cultivation medium (3.1) are added. The cells are re-suspended by gently inverting the tube and should not be pipetted up and down. This cell suspension is kept on ice until plating.

- Quality check by Trypan blue stain (see 4.3) and counting.

## Trypan blue stain for viability check and counting

- Prepare a 1:10 dilution of your cell suspension by adding 50 µL of cell suspension to 450 µL cultivation media. Invert the tube, but do not harshly pipette up and down. Place on ice.

- Prepare a 1:20 dilution of your cell suspension by gently mixing 50 µL of 0.4% Trypan blue solution with 50 µL of the 1:10 diluted cell suspension prepared before. Gently pipette up and down but avoid harsh pipetting.

- Quickly fill the 1:20 diluted Trypan blue cell suspension into a hemocytometer and count the amount of viable and dead (blue) cells. Cells in Trypan blue solution rapidly loose viability, therefore the counting should be performed directly after preparation of the 1:20 dilution mixture. The dilution process and the counting itself should not exceed more than 5 min duration in total.

- Count the cells under a bright field microscope by using a hemocytometer. Fill 10 µL of 1:20 diluted Trypan blue cell suspension into both chambers of the hemocytometer. Count cells in all 4 quartiles in both chambers (each quartile consists of 4x4 smaller squares, see Figure 1, marked in yellow) and note viable as well as dead blue cells. Calculate the average of viable cells per quartile and determine the cell viability as given below. Do not use hepatocytes with viability less than 60 %. In case the viability is higher than 60 % you can continue and plate the cells.

Figure 1: Example for counting cells with a Neubauer chamber (0.100 mm depth)

- Calculate the amount of vital cells per mL as follows:

Cells per mL =average living cells per quartile x dilution factor x 10.000

## Seeding primary human hepatocytes

Primary human hepatocytes are seeded in 6-well plates using 1 million cells per well in 2 mL full medium per well. After successful polymerization of the first layer of the collagen sandwich, 2 mL of full medium were given to each well (see 4.1). The following steps are performed to plate the cells:

- Calculate the amount of cell suspension containing 1 Mio of vital cells that needs to be pipetted to each well.

µL cell suspension containing 1 Mio cells=1000 µLvital cells per mL x 1,000,000

- Use cut tips to pipette the required amount of freshly isolated hepatocyte cell suspension to each well.

- Shake the plate gently side to side (in directions north-south; west-east) until cells are evenly distributed.

- Transfer plates in the incubator for at least 3 h of attachment. During the first 20 min of attachment, shake the cells as described every 5 min.

- After at least 3 h (maximum 5 h) of attachment, gently aspirate the medium and carefully add 2 mL of washing media to each well (3.3). Wash each well 3 times with washing media.

- Prepare appropriate amount of 1 mg/mL gelled collagen to be used as a second layer as described for the first layer (see 4.1, steps 1-3).

- Aspirate the washing media and add 350 µL of gelled collagen solution to each well. Distribute the gel by inverting the plate. The whole area of the well should be covered with collagen.

- Incubate the cells with the second layer of collagen for 45 min at 37°C ((incubator) to polymerize the gel.

- After 45 min of polymerization, add 2 mL of normal cultivation media to each well (3.1).

- Incubate the cells at 37°C overnight (16-20 h until treatment).

Start with treatments in the morning (between 8 and 12 o’clock) the day after plating.

SOP: PRIMARY HUMAN HEPATOCYTES RNA HARVESTING AND ISOLATION WITH TRIZOL/QIAZOL (Version 20160603)

# Equipment

- Bright field / Phase contrast microscope

- Pipet-aid, pipettes and micropipettes

- 2 mL tubes

- DNase and RNase free 1.5 mL tubes

- Cell scraper

- Ice box

- Sonicator (Finger)

- Micro centrifuge with cooling function

- Fume hood

# Substances

- Qiazol Lysis Reagent from Qiagen Sciences, Cat. No 79306

- Chloroform (Carl Roth, Karlsruhe, Germany, Cat. No.: 7331.2)

- Ethanol (absolute); VWR chemicals. Cat. No. 20821.33

- 2-Propanol (Carl Roth, Karlsruhe, Germany, Cat. No: 7590.1)

- DEPC treated water (Thermo Fischer Scientific)

# Preparations before harvesting RNA

- Take bright field images of a representative area of each well in 10x and 20x magnification. Each 6-well plate is taken separately out of the incubator and should be transferred back to the incubator within maximum 5 min.

- Prepare ice and a beaker with cell scrapers (1 scraper per well).

- Harvesting RNA

- Pick the cells from the incubator and directly transfer the plate on ice. Successively perform steps 1-5 for each plate.

- Quickly aspirate the medium.

- Place the plate on ice under a fume hood and add 1 mL of Qiazol to each well (6-well format).

- Lyse the cells by moving a cell scraper throughout each well. Scraping the cells also destroys the collagen sandwich. Use a fresh cell scraper for each well.

- As soon as all wells of a plate are treated and lysed with Qiazol, transfer the lysate of each well to a fresh 2 mL tube on ice.

- Sonicate each sample with a sonication finger while constantly keeping the samples on ice. Sonicate each sample for 30 sec (alternating 5 sec pulse, 2 sec break). Clean the sonication finger with distilled water and ethanol after each sample.

- If samples are not directly processed for RNA isolation, store Qiazol cell lysates at minus 80°C until further processing.

# Preparing work for RNA isolation

- Pre-cool the micro centrifuge to 4°C

- 100% and 75% (v/v) Ethanol in distilled water

- Fill aliquots (e.g. 50 mL polystyrene tubes) with isopropanol, 100 % and 75 % ethanol and chloroform

- Prepare are box with ice

# RNA isolation

- Defrost Qiazol-RNA samples on ice.

- Place samples in a rack for 2 mL tubes and transfer to a fume hood. Quickly add 200 µL chloroform to each sample.

- Strongly shake the samples for 15 sec. This can either be done by vortexing or by fixing the rack with the micro tubes with a second rack on top and shaking by hand.

- Incubate for 2-3 min at room temperature. A first phase separation should become visible.

- Centrifuge the samples at 12,000 x g and 4°C for 15 min. Use this time window to prepare a fresh RNase free 1.5 mL tube with 500 µL 2-propanol for each sample.

- Carefully transfer the upper aqueous phase (which contains the RNA) of each sample to the corresponding tube with 2-propanol. Work under a fume hood. Make sure that nothing of the lower organic phase is transferred.

- Incubate at room temperature for 10 min

- Pellet the RNA by centrifugation at 12,000 x g and 4°C for 15 min.

- Carefully remove and discard the supernatant from the pelleted RNA.

- Washing with 100% ethanol: Directly add 1 mL of 100% EtOH (v/v) to each sample and briefly vortex.

- Pellet the RNA by centrifugation at 7,500 x g and 4°C for 5 min.

- Directly remove and discard the supernatant from the pelleted RNA.

- Washing with 75% ethanol: Directly add 1 mL of 75% EtOH (v/v) to each sample and briefly vortex.

- Pellet the RNA by centrifugation at 7,500 x g at 4°C for 5 min.

- Directly remove and discard the supernatant from the pelleted RNA. Start with a 1 mL tip and continue with a smaller tip to carefully remove all liquid from the sample.

- Air-dry the samples by placing the lid-opened tubes with pelleted RNA under a fume hood for 5-10 min. All liquid needs to evaporate but the pellet should not become completely dry. The pellet should still be white and not transparent.

- Re-suspend the RNA pellet by adding approximately 20 µL of DEPC treated water to each sample. Vortex and briefly spin down.

- Incubate on ice for at least 30 min before measuring the RNA concentration.

- Determine the RNA concentration. Ideally, concentrations are set to 100-2000 ng/µL. Use DEPC treated H2O to adjust the RNA concentration.

- Freeze samples at -80°C until further usage.

# Time frame

Parts of the standard operation procedure should not exceed a certain time limit. Table 1 gives an overview on time limits that should be maintained.

Table 1: Time limits for SOP RNA harvesting and isolation procedure

| Step | Time frame | Comments |
| --- | --- | --- |
| Bright field imaging | 5 min per plate | Until cells are transferred back to the incubator |
| Harvesting RNA steps 1-4 | 3 min per plate | From taking the cells out of the incubator until cells are lysed and scraped in Qiazol |
| RNA isolation steps 1-20 |  | Quickly process all steps, do not interrupt in-between or leave the samples at room temperature for longer time period. Directly process the samples when the centrifugation steps are completed (steps 5, 8, 11, 14). |
| RNA isolation step 1 | maximum 1-2 h | Thawing on ice until further sample processing |
| RNA isolation step 17 | 5-10 min | Until all liquid has evaporated. RNA pellets should be white, but not transparent and completely dry. |

SOP: cDNA SYNTHESIS (Version 20160603)

# Equipment

- Ice box

- Thermocycler

- Micro centrifuge with cooling function

- DNase and RNase free microtubes that fit into the thermocycler (0.2 mL)

- 1.5 and 0.5 mL micro tubes

# Substances

- High Capacity cDNA Reverse Transcription Kit from Applied Biosystems (4368813), stored at -20°C

- Diethyl pyrocarbonate (DEPC) treated H2O (Thermo Fischer Scientific)

# Experimental workflow

To reversely transcribe isolated RNA into cDNA the High Capacity cDNA Reverse Transcription Kit from Applied Biosystems is used. Depending on the amount of RNA that is available for each sample, 500 ng - 2 µg RNA are reversely transcribed into cDNA.

- Calculate the volume that corresponds to 2 µg RNA for each sample. If the amount of RNA is not sufficient, calculate the volume that corresponds to either 500 ng or 1 µg RNA. This defined amount of RNA needs to be diluted in DEPC treated water up to a final volume of 10 µL for each sample. Calculate the amount of water that is required to obtain 2 µg (1 µg or 500 ng) of RNA in a final volume of 10 µL.

- Defrost the RNA samples on ice.

- Preparation of mixture 1 (Table 1). Mix the appropriate amount of RNA with DEPC treated H2O up to a final volume of 10 µL per sample in a micro tube that fits into the thermocycler (0.2 mL tube). Place on ice until all samples are processed. Keep RNA constantly on ice.

- Preparation of mixture 2 (Table 1). Use the High Capacity cDNA Reverse Transcription Kit from Applied Biosystems to prepare a master mix according to Table 1. Defrost primers, 10x RT buffer and dNTPs for ~10 min at room temperature, but keep the reverse transcriptase enzyme on ice. The table gives the required volume for one sample. Calculate the volumes required for a master mix for (n+1) samples and mix the appropriate amount of 10x RT buffer, primers, dNTPs, reverse transcriptase and DEPC H2O. Place on ice.

Table 1: Reaction mixture for cDNA synthesis

| Compound | Volume per reaction |
| --- | --- |
| Mixture 1 | 10 µL |
| RNA | 500 ng – 2 µg |
| DEPC H2O | up to 10 µL |
| Mixture 2 | 10 µL |
| 10x RT buffer | 2 µL |
| Random primers | 2 µL |
| dNTPs | 0.8 µL |
| Reverse Transcriptase | 1 µL |
| DEPC H2O | 4.2 µL |
| Final volume | 20 µL |

- Pipette 10 µL of the master mix of mixture 2 to each mixture 1 sample and place samples on ice. The total volume for each sample is now 20 µL.

- Briefly spin down the tubes at 4°C to collect all liquid at the bottom of each tube.

- Place the samples into the thermocycler and run the program listed in Table 2. Pre-heat the lid of the thermos cycling block to 95°C before the cycling program itself starts.

Table 2: Conditions for the thermal cycling program

| Step | Temperature | Time |
| --- | --- | --- |
| Incubation | 25°C | 10 min |
| Reverse Transcription | 37°C | 120 min |
| Inactivation | 85°C | 5 sec |
|  | 4°C | hold |

- Transfer the reaction tubes to a centrifuge and briefly spin down the cDNA.

- Adjust the concentration of cDNA to a final concentration of 10ng/µL by adding the appropriate amount of DEPC treated H2O. Add 180 µL of DEPC treated water to 2 µg RNA samples, 80 µL DEPC treated water to 1 µg RNA samples and 30 µL DEPC treated water to 500 ng RNA samples.

- Store the cDNA at 4°C for short term storage (up to one week) or for long time storage at minus 20°C until further usage.

# Time frame

The exact duration for cDNA synthesis depends on the number of samples that need to be processed.

Quickly process steps 3-7, do not interrupt in between or leave the samples at room temperature for longer time period. Finished cDNA is stored at 4°C for short time storage.

SOP: QUANTITATIVE REAL TIME PCR (Version 20160603)

# Equipment

- 7500 Real-Time PCR System, Applied Biosystems

- MicroAmp Optical 96-Well Reaction Plates (Applied Biosystems, Ref. N8010560)

- Rack for 1.5 mL tubes

- 96-well plate rack

- Pipet-aid, pipettes and micropipettes with tips

- DNase and RNase free 1.5 mL microtubes

- DNAZapTM (Invitrogen)

- Ice box

# Substances

- DEPC (Diethyl pyrocarbonate) treated H2O (Invitrogen)

- Taqman probes (Table 2)

- Universal PCR Master Mix II with UNG (Applied Biosystems, Part. No.: 4440038)

# Technical information

A reaction mixture is prepared according to following table. The volumes are for one sample.

Table 1: Reaction mix for qPCR				Table 2: Taqman probes for 7-gene signature

| Component: | Volume |
| --- | --- |
| Universal PCR Master Mix | 10 µL |
| DEPC treated water | 6.5 µL |
| Taqman probe | 1 µL |
| 10 ng/μl cDNA | 2.5 µL |

| Gene | Probe-number |
| --- | --- |
| GAPDH | REF 4352934E |
| CYP1B1 | Hs00164383_m1 |
| CYP3A7 | Hs00426361_m1 |
| SULT1C2 | Hs00602560_m1 |
| FBXO32 | Hs01041408_m1 |
| TUBB2B | Hs00603550_g1 |
| G6PD | Hs00166169_m1 |
| RGCC | Hs00204129_m1 |
| UBC | Hs00824723_m1 |

Table 3: Thermal conditions for qPCR

| Stage | Temperature | Time | Repetitions |
| --- | --- | --- | --- |
| 1 | 50°C | 2 min | 1 |
| 2 | 95°C | 10 min | 1 |
| 3 | 94°C | 15 sec | 40-45 |
|  | 60°C | 30 sec |  |
|  | 72°C | 35 sec |  |
| 4 | 95°C | 15 sec | 1 |
|  | 60°C | 20 sec |  |
|  | 95°C | 15 sec |  |
|  | 60°C | 15 sec |  |

All samples were measured in technical duplicates. For each probe a negative control was prepared by substituting the cDNA with DEPC water. The technical settings are given in table 3.

# Workflow

Before starting you work: Put your samples, Taqman probes and universal PCR master mix on ice for thawing. Spray DNAZap solution 1 and DNAZap solution 2 on the working area and let it work for ~10 min. Afterwards use a paper towel to clean the surface.

Caution to prevent contaminations: Use RNase and DNase free filter tips. Place the 96-well plate on a rack. Wear gloves and a clean lab coat.

- Generate a master mix by pipetting the Taqman probe and the Universal PCR master mix together with DEPC treated water accordant to Table 1. (Calculate how much you need for all you samples)

- Pipette 2.5 µL cDNA carefully into a well of the 96-well plate

- Add 7.5 µL of the master mix into the corresponding well

Note: It’s easier if you pipette first all cDNA samples and then the corresponding master mix

- Seal the 96-well plate with a seal (suitable for qRT-PCR) and centrifuge the plate at 1000g for 1 min at room temperature

Note: Check for air bubbles. Centrifuge again if necessary

- Place the plate into the real time PCR system and start the program listed in Table 3.

SOP: HEPG2 CULTIVATION AND PASSAGING (Version 20190123)

# Equipment

- HepG2 cultivation medium

- Incubator (water jacketed, 37°C, humidified atmosphere of air containing 5% CO2)

- Temperature-controlled bath (37°C)

- Laminar airflow cabinet

- Centrifuge

# Substances

- Trypsin/EDTA (Ethylene Diamine Tetraacetic Acid) (Sigma)

- PBS (Phosphate Buffered Saline)

# Medium

Table 1: Components of HeG2 cell culture medium

| Compound | Company | Catalog number |
| --- | --- | --- |
| Dulbecco's modified eagle's medium (DMEM) | PAN Biotech GmbH, Aidenbach, Germany | P04-04500 |
| Penicillin/Streptomycin | PAN Biotech GmbH, Aidenbach, Germany | P06-07100 |
| Sera Plus (Special Processed FBS) | PAN Biotech GmbH, Aidenbach, Germany | 3702-P103009 |

Table 2: Composition of HepG2 cell culture medium

| Compound | Volume |
| --- | --- |
| Dulbecco's modified eagle's medium (DMEM) | 500 mL |
| Penicillin/Streptomycin | 5 mL |
| Sera Plus (Special Processed FBS | 50 mL |

Note: Prior to preparing the HepG2 culture medium the FCS is heat inactivated at 56°C for 30 min. Always pre-heat the medium at 37°C.

# Thawing of HepG2 cells

The frozen aliquots of HepG2 cells are quickly thawed in a 37°C warm water bath before being transferred in a 50 mL centrifugation tube containing 9 mL of cold HepG2 culture medium. The cell suspension then is centrifuged for 5 min at 600 rpm at room temperature. The supernatant is removed, 10 mL of cold PBS are added and the cell suspension is again centrifuged for 5 min at 600 rpm at room temperature. After removing the supernatant the pellet is resuspended in 10 mL of warm HepG2 culture medium and transferred into a T75 cell culture flask.

# Cultivation of HepG2 cells

HepG2 are cultivated in T75 or T175 flasks (Sarstedt) at 37°C, 5% CO2 and constant humidity.

Note: Prior to preparing the HepG2 culture medium the FCS is heat inactivated at 56°C for 30 min. Always pre-heat the medium at 37°C.

# Passaging of HepG2 cells

For subculturing of HepG2 cells are passaged when 80-90% confluence is reached. First the medium is removed, then the cells are washed one time with 10-15 mL of warm PBS. Afterwards 2 mL (for a T75 flask) or 4 mL (for a T175 flask).of Trypsin/EDTA are added-. The cells are incubated for 7-8 min at 37°C and 5% CO2. Afterwards 8 mL (in case of T75 flask) or 16 mL (in case of T175) of warm HepG2 culture medium are added, the cell suspension is transferred into a 50 mL centrifugation tube and centrifuged for 5 min at 600 rpm and room temperature. The supernatant is removed and the pellet is resuspended in 1 mL of warm HepG2 culture medium by repeatedly pipetting up and down with a 1000 µL pipette tip. Afterwards the cell suspension is diluted with warm HepG2 cell culture medium and distributed into new cell culture flasks.

SOP: HepG2 CELLS CYTOTOXICITY TESTING WITH CELL-TITER-BLUE VIABILITY ASSAY (Version 20170720)

Cytotoxicity tests (CellTiter-Blue viability assays) are performed in three independent experiments, meaning three biological replicates with HepG2 cells of different passages. Cells are cultivated in conventional 24-well plates, seeding 62,500 cells per well.

# Equipment

- Laminar airflow cabinet

- Incubator (water jacketed, 37°C, humidified atmosphere of air containing 5% CO2)

- Temperature-controlled bath (37°C)

- Bright field microscope

- Cell culture microplate 96-well, black (Greiner bio-one, Frickenhausen, Germany, REF 655986)

- Infinite M200 Pro Plate reader (Tecan)

# Substances

- Cell Titer Blue Reagent from Promega (Cat. No. G8081; stored in aliquots at -20°C, thawed in the water bath before usage)

- Cultivation medium (DMEM 4.5% Glucose + 1% Penicillin/Streptomycin + 10% heat inactivated FBS, see Table 1)

- Test compounds

Table 1: Medium components for HepG2 cell cultivation

| Compound | Company | Catalog number |
| --- | --- | --- |
| Dulbecco's modified eagle's medium (DMEM) | PAN Biotech GmbH, Aidenbach, Germany | P04-04500 |
| Penicillin/Streptomycin | PAN Biotech GmbH, Aidenbach, Germany | P06-07100 |
| Sera Plus (Special Processed FBS) | PAN Biotech GmbH, Aidenbach, Germany | 3702-P103009 |

# Time schedule and general overview

Compound treatment starts 16-20 h after plating cells. Five concentrations per compound plus solvent control are tested for each compound and cells are exposed for 48 h. After the duration of incubation each well is briefly considered under the microscope and a viability score is estimated on a scale from 1 - 10 (10 = all cells vital, 1 = all cells dead).

Afterwards, the medium is aspirated and fresh medium with Cell Titer Blue reagent is added to the cells. After 2 h of incubation at 37°C, the supernatant of each well is transferred to a 96-well plate and the fluorescence intensity is read out at 594 nm using the Tecan Infinite M200 Pro plate reader (i-control software (version 1.7.1.12)).

Figure 1: Time schedule and general overview for cytotoxicity tests in HepG2 cells

# Compound treatment

Compound exposure starts between 16 and 20 h after plating the cells. Each compound is tested in 5 different concentrations (C1 (lowest) to C5 (highest conc.)) plus vehicle control. The vehicle control corresponds to the solvent concentration in the highest dose (C5). All five concentrations and the solvent control are tested on the same 24-well plate. Figure 2 shows exemplary the plate set up for 4 compounds (compounds A, B, X and Y).

Figure 2: Plate set up for cytotoxicity tests in HepG2 cells

Each well of the 24-well plate requires 0.5 mL of medium with the appropriate compound concentration or solvent control.

- Compounds that are soluble in water/medium are used to freshly prepare the solutions C1 to C5 on the day of treatment. Start your dilution series with the highest concentration (C5). Prepare the highest concentration (C5) by dissolving the compound in medium. Mix well by vortexing. If necessary, place the solution in the water bath at 37°C. Prepare C4 by mixing fresh cultivation medium with an appropriate amount of the highest concentration (C5). Continue the dilution series with the remaining concentrations, always by mixing medium with the appropriate volume of the next higher concentration. Therefore shake or vortex the next higher concentration before preparing the corresponding lower concentrated solution.

- In case your compound is insoluble in medium itself, prepare stock solutions of your test compound in DMSO. We usually use 0.1% DMSO as final DMSO concentration, therefore we prepare 1000x higher concentrated stock solutions. Such stock solutions are prepared for all concentrations of one compound to harmonize and adjust the final DMSO concentration for all conditions. If compounds are at the limit of solubility (for eg stock solution C5), 200x higher concentrated stock solutions are used for C5 or C5 and C4 (final DMSO concentration is 0.5%).

Start your dilution series with the highest concentration (C5). Prepare each concentration by diluting the corresponding stock solution in medium. Mix well by vortexing. If necessary, place your stock solution in the water bath at 37°C to support dissolving the compound.

- When all dilution series are prepared, pre-warm the solutions in the water bath (37°C).

- Collect each plate separately from the incubator and transfer under the sterile hood (laminar airflow cabinet).

- Gently aspirate the medium from all wells. Then add 0.5 mL of the appropriate solution to each well, starting with the solvent control and continue with increasing concentrations. Shake or vortex each concentration directly before adding it to the cells. This step should not take longer than 4 min.

- Place the cells back into the incubator and incubate for 48 h.

- After 46-47 h of incubation, the cell morphology for each well is checked under the microscope. A viability score is assigned to each well on a scale from 1 – 10, matching a 10 for total viable and healthy cells and a 1 for cells which are dead. Consider each plate individually and keep the remaining plates in the incubator in between. After the morphology check place the cells back into the incubator until the 48 h of exposure are complete.

# Cell Titer Blue Assay

After 48 h of compound exposure, CellTiter-Blue (CTB) viability assays are performed. The assay is based on the metabolic capacity of cells and analyzes the reduction rate of the dark blue indicator dye resazurin to the pink and highly fluorescent dye resorufin. While resazurin is only slightly fluorescent, resorufin is highly fluorescent and can be quantified at its emission maximum at a wavelength of 584 nm. Vital cells exhibit a strong metabolization capacity for resazurin, and the fluorescence intensity in the medium supernatant is increasing. In contrast, non-viable cells show a decreased resazurin metabolization capacity, resulting in lower fluorescence intensity or, in case of dead cells the dark blue resazurin is not even metabolized.

Before the 48 h of exposure are complete, CTB-medium is prepared for all plates as follows: Each well requires a total amount of 0.6 mL fresh CTB-medium. 0.6 mL CTB-medium is composed of 0.1 mL CTB reagent and 0.5 mL normal cultivation medium. Prepare a master mix for x+2 wells: you will need (x+2) * 0.1 mL CTB reagent + (x+2) * 0.5 mL normal cultivation medium. CTB reagent is light sensitive, therefore the light in the laminar airflow cabinet should be turned off while working with CTB or CTB-medium.

- After 48 h of compound exposure, take out the plate and gently aspirate the medium of each well. Treat each plate separately.

- Subsequently add 0.6 mL of CTB-medium to each well (light turned off!). Note the time for each plate. Leave 5-10 min before treating the next plate with CTB-medium.

- Incubate for 2 h at 37°C. During this incubation time the dark blue indicator dye resazurin is metabolized to the pink and highly fluorescent dye resorufin. The color of the control medium will shift from blue (resazurin) to pink (resorufin) whereas the compound-exposed wells should ideally exhibit a color gradation from pink (C1) to blue (C5).

- The read out is performed in black bottomed 96-well plates (Greiner bio-one), therefore the supernatant medium of each well needs to be transferred from the 24-well plates to 96-well plates. Work in the dark. Collect each plate individually from the incubator and place the cells on the normal work space area (unsterile working environment). The fluorescence value for each well is determined as a triplicate, therefore 3 x 100 µL of each well are quickly transferred to 3 wells in the 96-well plate (see Figure 3). Include at least 1 well of fresh CTB-medium that was not in contact with cells. This value is important for the background fluorescence.

Figure 3: Pipetting scheme for 96 well plates for CellTiter-Blue read out

Transferring the supernatant from a 24-well plate well to the 96-well plate should not take longer than 5 min for each plate. It is important to separate the supernatant from the cells, otherwise the remaining viable cells will metabolize remaining blue dye (resazurin) and all wells, independent of the compound concentration, will exhibit the pinkish color (resorufin) as seen in the solvent control.

- When all supernatants are transferred to the 96-well plate, the plate can be either read out immediately (see step 6) or kept aside in the dark for 20-30 min (until all remaining 96-well plates for the other compounds are complete).

- Use the Tecan Infinite M200 Pro plate reader (software i-control, version 1.7.1.12): Excitation wavelength 540 nm, emission wavelength 594 nm. The output is a fluorescence value for each well of the 96 well plate.

# Calculations

- Subtract the background fluorescence from each measurement. If CTB background control was measured repeatedly, calculate the mean value and subtract this value from each well.

- Untreated cells (vehicle controls) are used as reference for 100% viability. Calculate the cell viability as percentage of control. Include standard deviation and standard error of the mean.

SOP: HepG2 CELLS COLLECTION OF SAMPLES FOR GENE EXPRESSION ANALYSIS (Version 20190123)

# Equipment

- HepG2 cultivation medium

- Incubator (water jacketed, 37°C, humidified atmosphere of air containing 5% CO2)

- Temperature-controlled bath (37°C)

- Laminar airflow cabinet

- Centrifuge

- CASY  cell counter

- 6-well cell culture plates (Sarstedt)

- Cell scraper

- 1.5 mL reaction cubes

- Ice bath

- Sonicator

- Filter pipette tips (1000 µL)

- Chemical hood

- Microscope with camera

# Substances

- Trypsin/EDTA (Ethylene Diamine Tetraacetic Acid) (Sigma)

- PBS (Phosphate Buffered Saline)

- HepG2 cultivation medium

# Plating of cells

After reaching of 80-90% confluency in the cell culture flask the cells are plated for gene expression analysis. First the medium is removed, then the cells are washed one time with 10-15 mL of warm PBS. Afterwards 2 mL (for a T75 flask) or 4 mL (for a T175 flask).of Trypsin/EDTA are added-. The cells are incubated for 7-8 min at 37°C and 5% CO2. Afterwards 8 mL (in case of T75 flask) or 16 mL (in case of T175) of warm HepG2 culture medium are added, the cell suspension is transferred into a 50 mL centrifugation tube and centrifuged for 5 min at 600 rpm and room temperature. The supernatant is removed and the pellet is resuspended in 1 mL of warm HepG2 culture medium by repeatedly pipetting up and down with a 1000 µL pipette tip. Afterwards the cell suspension is diluted by adding 11 – 14 mL of warm HepG2 culture medium and thoroughly resuspended.

100 µL of cell suspension is transferred into a casy cup containing 10 mL of casyton. The cells are then counted with a Casy cell counter (Innovartis). The cell suspension is after a further thorough resuspension diluted to a concentration of 500,000 vital cells/ 2 mL of medium. 2 mL of this suspension is plated/well in a 6-well cell culture plate. The cells are kept overnight at 37°C and 5% CO2.

# Treatment of cells

On the next morning the treatment solutions of the compounds and solvent controls in HepG2 are freshly prepared in warm HepG2 cultivation medium. Each well receives 2 mL of treatment solution or solvent control. The cells are incubated for 24 h at 37°C and 5% CO2.

# Collection of samples for RNA extraction

Prior to the end of the 24 h incubation period pictures of each well are taken with 10x and 20x magnification. After 24 h of incubation the cell culture plate is placed on ice, the medium is quickly removed and 1 mL of Qiazol reagent is added to each well under the fume hood. The cells are lysed by moving the cell scraper through the well in all directions. The cell lysate is then transferred into 1.5 mL reaction tubes and sonicated with sonification finger for 30 sec with alternating 5 sec pulse and 2 sec break. During the sonification step the sample is constantly kept on ice.

Either the RNA was extracted immediately or the samples were stored at -80°C.

SOP: HepG2 CELLS GLUTATHION DEPLETION ASSAY (Version 20170314)

Glutathion depletion assays are performed in three independent experiments, meaning three biological replicates with HepG2 cells of different passages. Cells are cultivated in black 96-well plates with transparent bottom, seeding 15,625 cells in 200 µL medium per well.

!!!! Leave at least three wells empty, where no cells are plated but which are later on also incubated with Dibromobimane solution. This is for background correction.

# Equipment

- Laminar airflow cabinet

- Incubator (water jacketed, 37°C, humidified atmosphere of air containing 5% CO2)

- Temperature-controlled bath (37°C)

- Cell culture microplate 96-well, black (Greiner bio-one, Frickenhausen, Germany, REF 655986)

- Infinite M200 Pro Plate reader (Tecan)

# Substances

- Cultivation medium (DMEM 4.5% Glucose + 1% Penicillin/Streptomycin + 10% heat inactivated FBS, see Table 1)

- Hoechst stock solution 0.5 mg/mL

- Compounds listed in Table 2

- PBS (Phosphate Buffered Saline)

Table 1: Medium components for HepG2 cell cultivation

| Compound | Company | Catalog number |
| --- | --- | --- |
| Dulbecco's modified eagle's medium (DMEM) | PAN Biotech GmbH, Aidenbach, Germany | P04-04500 |
| Penicillin/Streptomycin | PAN Biotech GmbH, Aidenbach, Germany | P06-07100 |
| Sera Plus (Special Processed FBS) | PAN Biotech GmbH, Aidenbach, Germany | 3702-P103009 |

Table 2: Reagents for GSH depletion assay

| Compound | Company | Catalog number |
| --- | --- | --- |
| tert-Butylhydroquinone | Sigma Aldrich | 112941-100G |
| DL-Buthionine-sulfoximine | Sigma Aldrich | 19176-250MG |
| Dibromobimane | Sigma Aldrich | 34025-25MG |

# Time schedule and general overview

Compound treatment starts 16-20 h after plating cells. Fife concentrations per compound plus solvent control are tested for each compound and cells are exposed for 48 h. A positive (tert-Butyl hydroquinone) and a negative (Buthionine sulfoxime) control are included.

After 48 h of incubation the medium is removed by turning the plate onto paper towels and slightly beating the remaining liquid out of the wells. Medium with Hoechst (0.5 µg/mL) is added to stain the nuclei for normalization. Cells are incubated for 30 min before the fluorescence is determined with a plate reader.

Afterwards, Hoechst-medium is removed and the cells are washed 1x with PBS. Finally the cells are exposed to 40 µM Dibromobimane (DBB) solution for 30 min at 37°C and the fluorescence is determined with a plate reader.

Figure 1: Time schedule and general overview for GSH depletion assay in HepG2 cells

# Compound treatment

Compound exposure starts between 16 and 20 h after plating the cells. Each compound is tested in 5 different concentrations (C1 (lowest) to C5 (highest conc.)) plus vehicle control. The vehicle control corresponds to the solvent concentration in the highest dose (C5). All five concentrations and the solvent control are tested in triplicates on the same 96-well plate. Figure 2 shows exemplary the plate set up for 4 compounds (compounds A, B, C and D).

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | A_ctrl | A_C1 | A_C2 | A_C3 | A_C4 | A_C5 | B_ctrl | B_C1 | B_C2 | B_C3 | B_C4 | B_C5 |
| B | A_ctrl | A_C1 | A_C2 | A_C3 | A_C4 | A_C5 | B_ctrl | B_C1 | B_C2 | B_C3 | B_C4 | B_C5 |
| C | A_ctrl | A_C1 | A_C2 | A_C3 | A_C4 | A_C5 | B_ctrl | B_C1 | B_C2 | B_C3 | B_C4 | B_C5 |
| D | C_ctrl | C_C1 | C_C2 | C_C3 | C_C4 | C_C5 | D_ctrl | D_C1 | D_C2 | D_C3 | D_C4 | D_C5 |
| E | C_ctrl | C_C1 | C_C2 | C_C3 | C_C4 | C_C5 | D_ctrl | D_C1 | D_C2 | D_C3 | D_C4 | D_C5 |
| F | C_ctrl | C_C1 | C_C2 | C_C3 | C_C4 | C_C5 | D_ctrl | D_C1 | D_C2 | D_C3 | D_C4 | D_C5 |
| G | tBHQ | BSO |  |  |  |  |  |  |  |  |  |  |
| H | tBHQ | BSO |  |  |  |  |  |  |  |  |  |  |

Figure 2: Plate set up for cytotoxicity tests in HepG2 cells

Each well of the 96-well plate requires 0.2 mL of medium. tBHQ = tert-Butylhydroquinone induces glutathione synthesis and is used as a positive control whereas BSO = Buthionin sulfoxide inhibits glutathione and is used as a negative control.  Positive control: 5 mM tBHQ stock solution in PBS, final tBHQ concentration in medium is 25 µM. Negative control: 10 mM stock solution in PBS, final BSO solution in medium is 100 µM.

- Compounds that are soluble in water/medium are used to freshly prepare the solutions C1 to C5 on the day of treatment. Start your dilution series with the highest concentration (C5). Prepare the highest concentration (C5) by dissolving the compound in medium. Mix well by vortexing. If necessary, place the solution for 10 minutes in the water bath at 37°C. Prepare C4 by mixing fresh cultivation medium with an appropriate amount of the highest concentration (C5). Continue the dilution series with the remaining concentrations, always by mixing medium with the appropriate volume of the next higher concentration. Therefore shake or vortex the next higher concentration before preparing the corresponding lower concentrated solution.

- In case your compound is insoluble in medium itself, prepare stock solutions of your test compound in DMSO. We usually use 0.1% DMSO as final DMSO concentration, therefore we prepare 1000x higher concentrated stock solutions. Such stock solutions are prepared for all concentrations of one compound to harmonize and adjust the final DMSO concentration for all conditions. If compounds are at the limit of solubility (for eg stock solution C5), 200x higher concentrated stock solutions are used for C5 or C5 and C4 (final DMSO concentration is 0.5%).

Start your dilution series with the highest concentration (C5). Prepare each concentration by diluting the corresponding stock solution in medium. Mix well by vortexing. If necessary, place your stock solution for in the water bath at 37°C to support dissolving the compound.

- When all dilution series are prepared, pre-warm the solutions in the water bath (37°C).

- Collect each plate separately from the incubator and transfer under the sterile hood (laminar airflow cabinet).

- Remove the medium from all wells by turning the plate onto a paper towel. Then add 0.2 mL of the appropriate solution to each well, starting with the solvent control and continue with increasing concentrations. Shake or vortex each concentration directly before adding it to the cells. This step should not take longer than 4 min.

- Place the cells back into the incubator and incubate for 48 hrs.

# Glutathion depletion assay

Preparations: Hoechst stock solution in PBS, 5 mg/mL, sterile filtered (final Hoechst concentration is 5 µg/mL), Dibromobimane stock solution in DMSO, 40 mM (final DBB concentration is 40 µM)

Prepare Hoechst medium for all wells, you require 100 µL of 0.5 µg/mL Hoechst medium per well.

Prepare DBB solution for each well, you require 100 µL of 40 µM DBB solution in PBS per well.

- After 48 h of compound exposure remove the compound-medium by tapping the plate onto a paper towel.

- Add 100 µL of 0.5 µg/mL Hoechst medium per well and incubate at 37°C for 30 min to stain the nuclei.

- Read out the Hoechst fluorescence with a plate reader: excitation 340 nm, emission 450 nm.

- Remove the Hoechst medium and wash 1x with warm PBS.

- Add 100 µL 40 µM DBB solution per well and incubate for 30 min at 37°C. Include at least three empty wells (without cells) with DBB solution for back ground correction.

- Read out the DBB fluorescence with a plate reader: excitation 393 nm, emission 477 nm.

# Calculations

- Start with the DBB data. Use the fluorescence value of an empty well without cells for background correction. Subtract the background fluorescence from each measurement.

Value A = Fluorescence – background fluorescence

- Use Hoechst fluorescence data = value B.

Normalize the data by calculating Value A/ Value B = C.

- Use the normalized data (Value C) for the calculations: untreated cells (vehicle controls) are used as reference for 100% GSH activity. Calculate the GSH depletion as percentage of control. Include standard deviation and standard error of the mean.