Supplementary material

- Chemicals

|  | Grade, assay | CAS Number | Supplier |
| --- | --- | --- | --- |
| Antipyrine | analytical standard | 60-80-0 | Sigma Aldrich |
| Atenolol | ≥98% (TLC) | 29122-68-7 | Sigma Aldrich |
| Caffeine | ≥98 %, ReagentPlus®, for HPLC | 58-08-2 | Sigma Aldrich |
| Carbamazepine | ≥98 % | 298-46-4 | Sigma Aldrich |
| Ciprofloxacin | ≥98.0% (HPLC) | 85721-33-1 | Sigma Aldrich |
| Diclofenac |  | 15307-79-6 | Sigma Aldrich |
| Ketoprofen | ≥98% (TLC) | 22071-15-4 | Sigma Aldrich |
| Ofloxacin |  | 82419-36-1 | Sigma Aldrich |
| Sulfamethoxazole | analytical standard | 723-46-6 | Sigma Aldrich |
| Methanol | ≥99.9%, CHROMASOLV(R) | 67-56-1 | Sigma Aldrich |
| Acetonitrile | ≥99.9%, LC-MS CHROMASOLV® | 75-05-8 | Sigma Aldrich |
| Ammonium acetate | ≥99.0%, for HPLC | 631-61-8 | Sigma Aldrich |
| Ammonium hydroxide solution | ≥25% in H2O, eluent additive for LC-MS | 1336-21-6 | Sigma Aldrich |
| Formic acid | ~98% | 64-18-6 | Sigma Aldrich |
| Ammonium formate | ≥99.0% | 540-69-2 | Sigma Aldrich |

Milli-Q water was generated by Milli-Q Advantage A10 Water Purification System (Millipore, Schwalbach, Germany).

- Quantification with LC-MS/MS – Positive mode

Column: Kinetex 2.6 µm C18 100 Å, 150 x 3 mm, Phenomenex, Aschaffenburg, Germany

Flow rate: 250 µl/min

Injection volume: 25 µl

Solvents: (A) methanol, (B) 10 mM formic acid / ammonium formate (pH 3.2)

Gradient: 0-1 min A:B = 10:90; 1-28 min A:B = 100:0; 28-29 min A:B = 100:0; 29-31 min A:B = 10:90; 31-34 = 10:90

| Compound | MW | DP | EP | CEP | Q1 | Q3 | CE | CXP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Antipirine | 188.20 | 41 | 11.5 | 14.5 | 189.2 | 104.2 | 31 | 4 |
| Atenolol | 266.34 | 36 | 12 | 20 | 267.2 | 145.0 | 39 | 4 |
| Caffeine | 194.19 | 81 | 12 | 16 | 194.1 | 179.1 | 33 | 4 |
| Carbamazepine | 236.27 | 36 | 6 | 16 | 237.2 | 194.2 | 27 | 4 |
| Ciprofloxacin | 331.35 | 46 | 12 | 22 | 332.2 | 231.0 | 43 | 4 |
| Clofibrate | 242.70 | 26 | 12 | 14 | 243.1 | 115.1 | 13 | 4 |
| Ofloxacin | 361.37 | 36 | 12 | 20 | 362.2 | 261.1 | 31 | 4 |
| Sulfamethaxazole | 253.30 | 31 | 4.5 | 16 | 254.1 | 156.2 | 19 | 4 |

Source/Gas Parameters

| Parameter | TEM | CUR | IS | GS1 | GS2 | CAD |
| --- | --- | --- | --- | --- | --- | --- |
| Value | 350 | 25 | 1500 | 40 | 40 | 5 |

- Quantification with LC-MS/MS – Negative mode

Column: Kinetex 2.6 µm C18 100 Å, 150 x 3 mm, Phenomenex, Aschaffenburg, Germany

Flow rate: 250 µl/min

Injection volume: 25 µl

Solvents: (A) acetonitrile, (B) 5 mM ammonium acetate / ammonim hydroxide (pH 8)

Gradient: 0-15 min A:B = 0:100; 15-20 min A:B = 60:40; 20-25 min A:B = 100:0; 25-28 min A:B = 0:100

| Compound | MW | DP | EP | CEP | Q1 | Q3 | CE | CXP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Clofibric acid | 214.65 | -21 | -11.5 |  | 127 | 150 | -20 | -2 |
| Diclofnac | 296.15 | -16 | -8 |  | 293.9 | 250.1 | -12 | -4 |
| Ketoprofen | 257.31 | -31 | -9.5 |  | 252.9 | 209.2 | -8 | -4 |

Source/Gas Parameters

| Parameter | TEM | CUR | IS | GS1 | GS2 | CAD |
| --- | --- | --- | --- | --- | --- | --- |
| Value | 350 | 40 | -4500 | 40 | 40 | 3 |

- Limits of qualification of pharmaceuticals

| Compound | Limit of qualification [µg/L] |
| --- | --- |
| Antipirine | 1 |
| Atenolol | 1 |
| Caffeine | 2 |
| Carbamazepine | 1 |
| Ciprofloxacin | 1 |
| Diclofnac | 2 |
| Ketoprofen | 2 |
| Ofloxacin | 1 |
| Sulfamethaxazole | 2 |

- Water properties

| Sediment |  | G | V | E |
| --- | --- | --- | --- | --- |
| Sampling location |  | Neuherberg | Caissargues | Montiró (Ventalló) |
| X |  | 692507 | 596897 | 505706 |
| Y |  | 5344268 | 4840217 | 4666972 |
| Conductivity* | µS/cm | NA | 747 | 1337 |
| Temperature* | °C | NA | 15.2 | 16.6 |
| pH* | - | 7.5 | 6.9 | 7.3 |
| HCO3- | mg/L | 335.0 | 302.6 | 264.0 |
| Alk T | mg/L | NA | NA | 220 |
| SO42- | mg/L | 7.35 | 77.26 | 269 |
| Cl- | mg/L | 33.47 | 30.89 | 55 |
| NO3- | mg/L | 6.12 | 35.52 | 37 |
| NO2- | mg/L | Nd | Nd | Nd |
| NH4+ | mg/L | Nd | Nd | Nd |
| Ca2+ | mg/L | 86.46 | 122.98 | 175 |
| Mg2+ | mg/L | 17.46 | 6.80 | 23 |
| Na+ | mg/L | 19.97 | 17.38 | 33 |
| K+ | mg/L | 1.07 | 1.86 | 3.13 |
| F- | mg/L | NA | NA | 0.09 |
| PO43- | mg/L | Nd | NA | 0.01 |
| Br- | mg/L | Nd | 0.238 | 0.13 |
| SiO2 | mg/L | NA | NA | 10 |
| TOC | mg/L | 0.7 | NA | 0.37 |
| TN | mg/L | NA | NA | 9.01 |

Notation: * – parameters measured in field, Nd – not detected, NA – not analysed

- Sorption isotherms

Horizontal error bars indicate the standard deviation of the triplicates. Vertical error bars represent errors calculated basing on the error propagation methodology, as S was calculated from other measurements, which all have their uncertainty (JCGM 100:2008). When error bars are not visible, they are smaller than the symbol size.

  - Sediment G (technical coarse sand)

Antipyrine			Atenolol			Caffeine

Carbamazepine		Ciprofloxacin			Diclofenac

Ketoprofen			Ofloxacin			Sulfamethoxazole

  - Sediment V (sandy loam)

Antipyrine			Atenolol			Caffeine

Carbamazepine		Ciprofloxacin			Diclofenac

Ketoprofen			Ofloxacin			Sulfamethoxazole

  - Sediment E1 (sandy clay loam)

Antipyrine			Atenolol			Caffeine

Carbamazepine		Ciprofloxacin			Diclofenac

Ketoprofen			Ofloxacin			Sulfamethoxazole

  - Sediment E2 (clay)

Antipyrine			Atenolol			Caffeine

Carbamazepine		Ciprofloxacin			Diclofenac

Ketoprofen			Ofloxacin			Sulfamethoxazole

  - Sediment E3 (medium sand)

Antipyrine			Atenolol			Caffeine

Carbamazepine		Ciprofloxacin			Diclofenac

Ketoprofen			Ofloxacin			Sulfamethoxazole

- Correlation graphs between Freundlich sorption coefficient KF and logKOW of selected compounds

- Sorption partition coefficients Kd and KF

| Compound | Sediment | Kd [L/kg] | KF [(L/kg)η] | η | Solution type | Experim. type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Atenolol | coarse sand |  |  |  | groundwater | batch | this study |
| Antipyrine | coarse sand |  |  |  | groundwater | batch | this study |
| Carbamazepine | coarse sand |  |  |  | groundwater | batch | this study |
| Caffeine | coarse sand |  |  |  | groundwater | batch | this study |
| Ciprofloxacin | coarse sand |  |  |  | groundwater | batch | this study |
| Diclofenac | coarse sand |  |  |  | groundwater | batch | this study |
| Ketoprofen | coarse sand |  |  |  | groundwater | batch | this study |
| Ofloxacin | coarse sand |  |  |  | groundwater | batch | this study |
| Sulfamethoxazole | coarse sand |  |  |  | groundwater | batch | this study |
| Atenolol | sandy loam |  |  |  | groundwater | batch | this study |
| Antipyrine | sandy loam |  |  |  | groundwater | batch | this study |
| Carbamazepine | sandy loam |  |  |  | groundwater | batch | this study |
| Caffeine | sandy loam |  |  |  | groundwater | batch | this study |
| Ciprofloxacin | sandy loam |  |  |  | groundwater | batch | this study |
| Diclofenac | sandy loam |  |  |  | groundwater | batch | this study |
| Ketoprofen | sandy loam |  |  |  | groundwater | batch | this study |
| Ofloxacin | sandy loam |  |  |  | groundwater | batch | this study |
| Sulfamethoxazole | sandy loam |  |  |  | groundwater | batch | this study |
| Atenolol | sandy clay loam |  |  |  | groundwater | batch | this study |
| Antipyrine | sandy clay loam |  |  |  | groundwater | batch | this study |
| Carbamazepine | sandy clay loam |  |  |  | groundwater | batch | this study |
| Caffeine | sandy clay loam |  |  |  | groundwater | batch | this study |
| Ciprofloxacin | sandy clay loam |  |  |  | groundwater | batch | this study |
| Diclofenac | sandy clay loam |  |  |  | groundwater | batch | this study |
| Ketoprofen | sandy clay loam |  |  |  | groundwater | batch | this study |
| Ofloxacin | sandy clay loam |  |  |  | groundwater | batch | this study |
| Sulfamethoxazole | sandy clay loam |  |  |  | groundwater | batch | this study |
| Atenolol | clay |  |  |  | groundwater | batch | this study |
| Antipyrine | clay |  |  |  | groundwater | batch | this study |
| Carbamazepine | clay |  |  |  | groundwater | batch | this study |
| Caffeine | clay |  |  |  | groundwater | batch | this study |
| Ciprofloxacin | clay |  |  |  | groundwater | batch | this study |
| Diclofenac | clay |  |  |  | groundwater | batch | this study |
| Ketoprofen | clay |  |  |  | groundwater | batch | this study |
| Ofloxacin | clay |  |  |  | groundwater | batch | this study |
| Sulfamethoxazole | clay |  |  |  | groundwater | batch | this study |
| Atenolol | medium sand |  |  |  | groundwater | batch | this study |
| Antipyrine | medium sand |  |  |  | groundwater | batch | this study |
| Carbamazepine | medium sand |  |  |  | groundwater | batch | this study |
| Caffeine | medium sand |  |  |  | groundwater | batch | this study |
| Ciprofloxacin | medium sand |  |  |  | groundwater | batch | this study |
| Diclofenac | medium sand |  |  |  | groundwater | batch | this study |
| Ketoprofen | medium sand |  |  |  | groundwater | batch | this study |
| Ofloxacin | medium sand |  |  |  | groundwater | batch | this study |
| Sulfamethoxazole | medium sand |  |  |  | groundwater | batch | this study |
| Atenolol | sand | 1.54 |  |  | tap water | column | 1 |
| Antipyrine | sand | 0.00 |  |  | tap water | column | 1 |
| Carbamazepine | agricultural soil | 1.79 | 3.35 | 4.80 | 0.01 M CaCl₂ | batch | 2 |
| Carbamazepine | loamy fine sand |  | 12.63 | 0.87 | secondary-treated wastewater | batch | 3 |
| Carbamazepine | loamy fine sand |  | 11.46 | 1.21 | freshwater | batch | 3 |
| Diclofenac | loamy fine sand |  | 11.90 | 0.27 | secondary-treated wastewater | batch | 3 |
| Diclofenac | loamy fine sand |  | 8.81 | 0.49 | freshwater | batch | 3 |
| Carbamazepine | sandy loam |  | 0.87 | 0.91 | secondary-treated wastewater | batch | 3 |
| Carbamazepine | sandy loam |  | 1.21 | 0.73 | freshwater | batch | 3 |
| Diclofenac | sandy loam |  | 0.27 | 1.13 | secondary-treated wastewater | batch | 3 |
| Diclofenac | sandy loam |  | 0.49 | 0.92 | freshwater | batch | 3 |
| Carbamazepine | clay (low TOC) | 0.49 | 0.51 | 0.95 | 0.01 M CaCl₂ | batch | 4 |
| Carbamazepine | sandy loam (high TOC) | 37.00 | 57.00 | 0.67 | 0.01 M CaCl₂ | batch | 4 |
| Sulfamethoxazole | clay (low TOC) | 0.23 | 0.28 | 0.66 | 0.01 M CaCl₂ | batch | 4 |
| Sulfamethoxazole | sandy loam (high TOC) | 37.60 | 36.20 | 0.94 | 0.01 M CaCl₂ | batch | 4 |
| Ofloxacin | clay (low TOC) | 1192.00 | 832.00 | 0.61 | 0.01 M CaCl₂ | batch | 4 |
| Ofloxacin | sandy loam (high TOC) | 3554.00 | 3224.00 | 0.94 | 0.01 M CaCl₂ | batch | 4 |
| Diclofenac | clay (low TOC) | 0.45 | 0.44 | 1.00 | 0.01 M CaCl₂ | batch | 4 |
| Diclofenac | sandy loam (high TOC) | 164.50 | 172.00 | 0.92 | 0.01 M CaCl₂ | batch | 4 |
| carbamazepine | silty clay | 5.81 |  |  | 0.01 M CaCl₂ | batch | 5 |
| carbamazepine | silty clay | 9.11 |  |  | untreated wastewater | batch | 5 |
| carbamazepine | clay | 11.40 |  |  | 0.01 M CaCl₂ | batch | 5 |
| Carbamazepine | clay | 9.19 |  |  | untreated wastewater | batch | 5 |
| Carbamazepine | topsoil/loam | 4.61 | 12.40 | 0.87 | 0.01 M CaCl₂ | batch | 6 |
| Carbamazepine | agricultural soil/sand | 1.72 | 3.80 | 0.89 | 0.01 M CaCl₂ | batch | 6 |
| Atenolol | sand |  | 0.67 | 0.85 | 0.01 M CaCl₂ | batch | 7 |
| Carbamazepine | sand |  | 0.26 | 0.88 | 0.01 M CaCl₂ | batch | 7 |
| Sulfamethoxazole | sand |  | 0.11 | 0.61 | 0.01 M CaCl₂ | batch | 7 |
| Atenolol | loess |  | 5.08 | 0.85 | 0.01 M CaCl₂ | batch | 7 |
| Carbamazepine | loess |  | 0.67 | 0.88 | 0.01 M CaCl₂ | batch | 7 |
| Sulfamethoxazole | loess |  | 0.07 | 0.61 | 0.01 M CaCl₂ | batch | 7 |
| ciprofloxacin | soil (loamy sand) | 427.00 |  |  | 0.01 M CaCl₂ | batch | 8 |
| Ofloxacin | soil (loamy sand) | 309.00 |  |  | 0.01 M CaCl₂ | batch | 8 |
| Carbamazepine | medium sand | 5.30 | 5.34 | 0.99 | artificially synthetised groundwater | batch | 9 |
| Carbamazepine | fine sand | 0.21 | 0.13 | 1.08 | artificially synthetised groundwater | batch | 9 |
| Diclofenac | medium sand | 4.66 | 0.57 | 1.43 | artificially synthetised groundwater | batch | 9 |
| Carbamazepine | medium sand | 1.43 | 3.89 | 0.78 | artificially synthetised groundwater | batch | 9 |
| Diclofenac | medium sand | 1.87 | 0.81 | 1.18 | artificially synthetised groundwater | batch | 9 |
| Diclofenac | fine sand | 0.55 | 7.31 | 0.57 | artificially synthetised groundwater | batch | 9 |
| Carbamazepine | sand | 0.13 |  |  | synthesized treated sewage water | column | 10 |
| Diclofenac | sand | 0.57 |  |  | synthesized treated sewage water | column | 10 |
| Sulfamethoxazole | silt loam | 3.28 | 3.26 | 0.96 | 0.005 M CaCl₂ | batch | 11 |
| Sulfamethoxazole | silt loam | 3.36 | 6.10 | 0.79 | 0.005 M CaCl₂ | batch | 11 |
| Sulfamethoxazole | sit loam | 1.31 | 2.28 | 0.82 | 0.005 M CaCl₂ | batch | 11 |
| Carbamazepine | river sediment | 1.70 | 2.00 | 0.90 | 0.01 M CaCl₂ | batch | 12 |
| Carbamazepine | river sediment | 12.30 | 14.10 | 0.82 | 0.01 M CaCl₂ | batch | 12 |
| Ciprofloxacin |  | 150-45000 |  |  |  |  | 13 |
| Atnolol | river sediment | 1.30 |  |  | 0.01 M CaCl₂ | batch | 14 |
| Atnolol | river sediment | 8.10 |  |  | 0.01 M CaCl₂ | batch | 14 |
| Atnolol | river sediment | 5.30 |  |  | 0.01 M CaCl₂ | batch | 14 |
| Atnolol | loam silt | 1.10 |  |  | 0.01 M CaCl₂ | batch | 14 |
| Carbamazepine | river sediment | 8.50 |  |  | 0.01 M CaCl₂ | batch | 14 |
| Carbamazepine | river sediment | 1.40 |  |  | 0.01 M CaCl₂ | batch | 14 |
| Carbamazepine | river sediment | 1.80 |  |  | 0.01 M CaCl₂ | batch | 14 |
| Carbamazepine | loam silt | 1.20 |  |  | 0.01 M CaCl₂ | batch | 14 |
| Ofloxacin | natural soil | 75.80 | 1778.28 | 0.46 | CaCl₂ soultion | batch | 15 |
| Ofloxacin | soil (paddy field) | 46.60 | 1380.38 | 0.42 | CaCl₂ soultion | batch | 15 |
| Ofloxacin | soil (dry farmland) | 67.80 | 1698.24 | 0.45 | CaCl₂ soultion | batch | 15 |
| Ofloxacin | soil (pseudo-ginseng) | 250.00 | 3981.07 | 0.52 | CaCl₂ soultion | batch | 15 |
| Ofloxacin | soil (pseudo-ginseng) | 158.00 | 3630.78 | 0.46 | CaCl₂ soultion | batch | 15 |
| Ofloxacin | natural soil | 234.00 | 2041.74 | 0.63 | CaCl₂ soultion | batch | 15 |
| Ofloxacin | natural soil | 239.00 | 3715.35 | 0.53 | CaCl₂ soultion | batch | 15 |
| Ofloxacin | natural soil | 409.00 | 257.04 | 1.08 | CaCl₂ soultion | batch | 15 |
| Ofloxacin | soil (paddy field) | 264.00 | 1621.81 | 0.69 | CaCl₂ soultion | batch | 15 |
| Ofloxacin | agricultural soil | 156.00 | 467.74 | 0.81 | CaCl₂ soultion | batch | 15 |
| Ofloxacin | peat soil | 530.00 | 9332.54 | 0.51 | CaCl₂ soultion | batch | 15 |
| Ofloxacin | natural soil | 384.00 | 18620.87 | 0.34 | CaCl₂ soultion | batch | 15 |
| Ofloxacin | quartz | 1.30 | 26.30 | 0.48 | CaCl₂ soultion | batch | 15 |
| Ofloxacin | muscovite | 2.20 | 70.79 | 0.40 | CaCl₂ soultion | batch | 15 |
| Ofloxacin | kaolinite | 60.50 | 2238.72 | 0.38 | CaCl₂ soultion | batch | 15 |
| Ofloxacin | montmorillonite | 1510.00 | 95499.26 | 0.29 | CaCl₂ soultion | batch | 15 |

References

1.	Burke, V., Treumann, S., Duennbier, U., Greskowiak, J., Massmann, G., 2013. Sorption behavior of 20 wastewater originated micropollutants in groundwater - Column experiments with pharmaceutical residues and industrial agents. J. Contam. Hydrol. 154, 29–41. doi:10.1016/j.jconhyd.2013.08.001

2.	Calisto, V., Esteves, V.I., 2012. Adsorption of the antiepileptic carbamazepine onto agricultural soils. J. Environ. Monit. 14, 1597–1603. doi:10.1039/c2em10895j

3.	Chefetz, B., Mualem, T., Ben-Ari, J., 2008. Sorption and mobility of pharmaceutical compounds in soil irrigated with reclaimed wastewater. Chemosphere 73, 1335–1343. doi:10.1016/j.chemosphere.2008.06.070

4.	Durán-Álvarez, J.C., Prado-Pano, B., Jiménez-Cisneros, B., 2012. Sorption and desorption of carbamazepine, naproxen and triclosan in a soil irrigated with raw wastewater: Estimation of the sorption parameters by considering the initial mass of the compounds in the soil. Chemosphere 88, 84–90. doi:10.1016/j.chemosphere.2012.02.067

5.	Fenet, H., Mathieu, O., Mahjoub, O., Li, Z., Hillaire-Buys, D., Casellas, C., Gomez, E., 2012. Carbamazepine, carbamazepine epoxide and dihydroxycarbamazepine sorption to soil and occurrence in a wastewater reuse site in Tunisia. Chemosphere 88, 49–54. doi:10.1016/j.chemosphere.2012.02.050

6.	JCGM 100:2008, 2008. Evaluation of measurement data – Guide to the expression of uncertainty in measurement. Joint Committee for Guides in Metrology.

7.	Kodešová, R., Grabic, R., Kočárek, M., Klement, A., Golovko, O., Fér, M., Nikodem, A., Jakšík, O., 2015. Pharmaceuticals’ sorptions relative to properties of thirteen different soils. Sci. Total Environ. 511, 435–443. doi:10.1016/j.scitotenv.2014.12.088

8.	Nowara, A., Spiteller, M., 1997. Binding of Fluoroquinolone Carboxylic Acid Derivatives to Clay Minerals. J. Agric. Food Chem. 45, 1459–1463. doi:10.1021/jf960215l

9.	Scheytt, T., Mersmann, P., Lindstädt, R., Heberer, T., 2005. Determination of sorption coefficients of pharmaceutically active substances carbamazepine, diclofenac, and ibuprofen, in sandy sediments. Chemosphere 60, 245–253. doi:10.1016/j.chemosphere.2004.12.042

10.	Scheytt, T.J., Mersmann, P., Heberer, T., 2006. Mobility of pharmaceuticals carbamazepine, diclofenac, ibuprofen, and propyphenazone in miscible-displacement experiments. J. Contam. Hydrol. 83, 53–69. doi:10.1016/j.jconhyd.2005.11.002

11.	Srinivasan, P., Sarmah, A.K., Manley-Harris, M., 2013. Co-contaminants and factors affecting the sorption behaviour of two sulfonamides in pasture soils. Environ. Pollut. 180, 165–172. doi:10.1016/j.envpol.2013.05.022

12.	Stein, K., Ramil, M., Fink, G., Sander, M., 2008. Analysis and Sorption of Psychoactive Drugs onto Sediment. Environ. Sci. Technol. 42, 6415–6423. doi:10.1021/es702959a

13.	Vasudevan, D., Bruland, G.L., Torrance, B.S., Upchurch, V.G., MacKay, A. a., 2009. pH-dependent ciprofloxacin sorption to soils: Interaction mechanisms and soil factors influencing sorption. Geoderma 151, 68–76. doi:10.1016/j.geoderma.2009.03.007

14.	Yamamoto, H., Nakamura, Y., Moriguchi, S., Nakamura, Y., Honda, Y., Tamura, I., Hirata, Y., Hayashi, A., Sekizawa, J., 2009. Persistence and partitioning of eight selected pharmaceuticals in the aquatic environment: Laboratory photolysis, biodegradation, and sorption experiments. Water Res. 43, 351–362. doi:10.1016/j.watres.2008.10.039

15.	Zhou, D., Chen, B., Wu, M., Liang, N., Zhang, D., Li, H., Pan, B., 2014. Ofloxacin sorption in soils after long-term tillage: The contribution of organic and mineral compositions. Sci. Total Environ. 497–498, 665–670. doi:10.1016/j.scitotenv.2014.07.130