Sorption and biodegradation parameters of selected pharmaceuticals in laboratory column experiments

Marleen Schübla,b, Aleksandra Kiecakb,c, Katrin Hugb, Jutta Lintelmannd,e, Ralf Zimmermannd, Christine Stumppa,b*

1Institute for Soil Physics and Rural Water Management, University of Natural Resources and Life Sciences (BOKU), Vienna, Austria

2Institute of Groundwater Ecology, Helmholtz Zentrum München-German Research Center of Environmental Health (GmbH), Neuherberg, Germany

3Chair of Hydrogeology, Technical University of Munich, Munich, Germany

4 Joint Mass Spectrometry Centre of the research unit Comprehensive Molecular Analytics, Helmholtz Zentrum München-German Research Center of Environmental Health (GmbH), Neuherberg, Germany and the Chair of Analytical Chemistry, University of Rostock, Germany

5Research Unit Molecular Endocrinology and Metabolism, Helmholtz Zentrum München-German Research Center of Environmental Health (GmbH), Neuherberg, Germany

*Corresponding author at: Institute for Soil Physics and Rural Water Management, University of Natural Resources and Life Sciences (BOKU), Vienna, Austria.

Supplementary details

S1	Tandem mass spectrometric determination of pharmaceutics

Instrumentation

Ultrahigh performance liquid chromatography (UHPLC): Agilent 1290 Infinity LC-system consisting of an autosampler 1290 G4226A, a pump 1290 G4220A and a column thermostat 1290 G1316C (Agilent Technologies, Santa Clara, US).

Mass spectrometer (MS): API 4000 QTrap from Sciex (Darmstadt, Germany).

Chromatographic separation

Analytical column:	Acquity HSS T3, 1,8 µm, 50 x 2,1 mm I.D.

Flow rate:	0.5 ml/min

Temperature: 20°C

Injection volume: 1 µl

Eluents: 	A: Formic acid/ammoniumformate (10 mmol/l, respectively), B: methanol

Gradient: 	From 5 % B to 95 % B in 4.5 min; to 100 %  B in 0.1 min. At 100 % B for 1.4 min. In 0.1 min to 5 % B again. These conditions are kept constant for 0.9 min (equilibration).

| Substance | m/z Q1 | m/z Q3 | DP (V) | CE (V) | CXP (V) | Dwelltime (ms) |
| --- | --- | --- | --- | --- | --- | --- |
| Caffeine | 195.2 | 110.1 | 72.0 | 33.0 | 9.0 | 50 |
| Caffeine | 195.2 | 179.1 | 75.0 | 36.0 | 9.0 | 50 |
| Antipirine | 189.2 | 104.1 | 75.0 | 34.0 | 6.0 | 50 |
| Antipirine | 189.2 | 161.2 | 74.0 | 25.0 | 9.0 | 50 |
| SMX | 254.1 | 156.2 | 63.0 | 23.0 | 9.0 | 50 |
| Atenolol | 267.2 | 190.2 | 70.0 | 27.0 | 9.0 | 50 |
| Atenolol | 267.2 | 145.0 | 71.0 | 37.0 | 9.0 | 50 |
| Carbamazepine | 237.2 | 194.1 | 70.0 | 28.0 | 9.0 | 50 |
| Carbamazepine | 237.2 | 220.2 | 69.0 | 21.0 | 9.0 | 50 |

DP: Declustering potential
CE: Collision energy
CXP: Cell exit potential

Ion spray voltage was 5500 V, source temperature was 650 °C.  Ion source gas 1 and ion source gas 2 were adjusted at 50 psi, collision gas had 11 psi, and curtain gas 20 psi.

S2	Ion chemistry of the groundwater used in this study: fresh groundwater (Neuherberg) and water at the outlet of G and V columns after one month of equilibration at a constant flow rate of q ≈ 0.5 mL/min (mean and standard deviation from triplet samples).

|  | Fresh groundwater [mg/L] | G columns [mg/L] | V columns [mg/L] |
| --- | --- | --- | --- |
| Li+ | 0.00 ± 0.00 | 0.00 ± 0.00 | 0.00 ± 0.00 |
| Na+ | 18.43 ± 0.12 | 18.78 ± 1.25 | 18.19 ± 0.3 |
| NH4+ | 0.00 ± 0.00 | 0.08 ± 0.03 | 0.00 ± 0.00 |
| K+ | 1.01 ± 0.00 | 1.05 ± 0.02 | 1.47 ± 0.39 |
| Mg2+ | 17.10 ± 0.10 | 17.04 ± 0.15 | 15.9 ± 0.88 |
| Ca2+ | 84.63 ± 0.47 | 75.18 ± 3.39 | 76.75 ± 3.58 |
| F- | 0.00 ± 0.00 | 0.00 ± 0.00 | 0.00 ± 0.00 |
| Cl- | 32.57 ± 0.06 | 33.92 ± 2.8 | 32.96 ± 0.47 |
| NO2- | 0.00 ± 0.00 | 0.00 ± 0.00 | 0.00 ± 0.00 |
| Br- | 0.00 ± 0.00 | 0.00 ± 0.00 | 0.00 ± 0.00 |
| NO3- | 6.87 ± 0.06 | 6.57 ± 0.38 | 6.17 ± 0.01 |
| HPO42- | 0.00 ± 0.00 | 0.00 ± 0.00 | 0.00 ± 0.00 |
| SO42- | 7.97 ± 0.06 | 8.30 ± 0.41 | 8.29 ± 0.06 |

S3	Nitrate and dissolved oxygen (DO) concentrations over time in outflow samples of all six column experiments